1. Meta Feature Types

1.1. Vertical Datum of Data

Definition: Any level surface (for example Mean Sea Level) taken as a surface of reference to which the elevations within a data set are reduced. Also called datum level, reference level, reference plane, levelling datum, datum for heights.

CamelCase: VerticalDatumOfData

Alias: M_VDAT

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Vertical Datum | (VERDAT) (Datum Level) (Reference Plane) (Levelling Datum) (Datum for Sounding Reduction) (Datum for Heights) | 1 : Mean Low Water Springs 2 : Mean Lower Low Water Springs 3 : Mean Sea Level 4 : Lowest Low Water 5 : Mean Low Water 6 : Lowest Low Water Springs 7 : Approximate Mean Low Water Springs 8 : Indian Spring Low Water 9 : Low Water Springs 10 : Approximate Lowest Astronomical Tide 11 : Nearly Lowest Low Water 12 : Mean Lower Low Water 13 : Low Water 14 : Approximate Mean Low Water 15 : Approximate Mean Lower Low Water 16 : Mean High Water 17 : Mean High Water Springs 18 : High Water 19 : Approximate Mean Sea Level 20 : High Water Springs 21 : Mean Higher High Water 22 : Equinoctial Spring Low Water 23 : Lowest Astronomical Tide 24 : Local Datum 25 : International Great Lakes Datum 1985 26 : Mean Water Level 27 : Lower Low Water Large Tide 28 : Higher High Water Large Tide 29 : Nearly Highest High Water 30 : Highest Astronomical Tide 31 : Local Low Water Reference Level 32 : Local High Water Reference Level 33 : Local Mean Water Reference Level 34 : Equivalent Height of Water (German GlW) 35 : Highest Shipping Height of Water (German HSW) 36 : Reference Low Water Level According to Danube Commission 37 : Highest Shipping Height of Water According to Danube Commission 38 : Dutch River Low Water Reference Level (OLR) 39 : Russian Project Water Level 40 : Russian Normal Backwater Level 41 : Ohio River Datum 43 : Dutch High Water Reference Level 44 : Baltic Sea Chart Datum 2000 45 : Dutch Estuary Low Water Reference Level (OLW) 46 : International Great Lakes Datum 2020 47 : Sea Floor 48 : Sea Surface 49 : Hydrographic Zero | EN | 1, 1 |

1.2. Data Coverage

Definition: A geographical area that describes the coverage and extent of spatial objects.

CamelCase: DataCoverage

Alias: M_COVR M_CSCL

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Maximum Display Scale |  |  | IN | 1, 1 |
| Minimum Display Scale |  |  | IN | 1, 1 |

1.3. Local Direction of Buoyage

Definition: An area within which the navigational system of marks has been established in relation to a specific direction.

CamelCase: LocalDirectionOfBuoyage

Alias: M_NSYS

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Orientation |  |  | C | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |

1.4. Navigational System of Marks

Definition: An area within which the navigational system of marks has been established in relation to a specific direction.

CamelCase: NavigationalSystemOfMarks

Alias: M_NSYS

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 1, 1 |

1.5. Sounding Datum

Definition: The horizontal plane or tidal datum to which soundings have been reduced. Also called datum for sounding reduction.

CamelCase: SoundingDatum

Alias: M_SDAT

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Vertical Datum | (VERDAT) (Datum Level) (Reference Plane) (Levelling Datum) (Datum for Sounding Reduction) (Datum for Heights) | 1 : Mean Low Water Springs 2 : Mean Lower Low Water Springs 3 : Mean Sea Level 4 : Lowest Low Water 5 : Mean Low Water 6 : Lowest Low Water Springs 7 : Approximate Mean Low Water Springs 8 : Indian Spring Low Water 9 : Low Water Springs 10 : Approximate Lowest Astronomical Tide 11 : Nearly Lowest Low Water 12 : Mean Lower Low Water 13 : Low Water 14 : Approximate Mean Low Water 15 : Approximate Mean Lower Low Water 16 : Mean High Water 17 : Mean High Water Springs 18 : High Water 19 : Approximate Mean Sea Level 20 : High Water Springs 21 : Mean Higher High Water 22 : Equinoctial Spring Low Water 23 : Lowest Astronomical Tide 24 : Local Datum 25 : International Great Lakes Datum 1985 26 : Mean Water Level 27 : Lower Low Water Large Tide 28 : Higher High Water Large Tide 29 : Nearly Highest High Water 30 : Highest Astronomical Tide 31 : Local Low Water Reference Level 32 : Local High Water Reference Level 33 : Local Mean Water Reference Level 34 : Equivalent Height of Water (German GlW) 35 : Highest Shipping Height of Water (German HSW) 36 : Reference Low Water Level According to Danube Commission 37 : Highest Shipping Height of Water According to Danube Commission 38 : Dutch River Low Water Reference Level (OLR) 39 : Russian Project Water Level 40 : Russian Normal Backwater Level 41 : Ohio River Datum 43 : Dutch High Water Reference Level 44 : Baltic Sea Chart Datum 2000 45 : Dutch Estuary Low Water Reference Level (OLW) 46 : International Great Lakes Datum 2020 47 : Sea Floor 48 : Sea Surface 49 : Hydrographic Zero | EN | 1, 1 |

1.6. Quality of Bathymetric Data

Definition: An area within which a uniform assessment of the quality of the bathymetric data exists.

CamelCase: QualityOfBathymetricData

Alias: M_QUAL

Super type:

Feature use type: meta

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Temporal Variation |  | 1 : Extreme Event 2 : Likely to Change and Significant Shoaling Expected 3 : Likely to Change But Significant Shoaling Not Expected 4 : Likely to Change 5 : Unlikely to Change 6 : Unassessed | EN | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Horizontal Distance Uncertainty | (HORACC) |  | (S) RE | 0, 1 |
| Horizontal Position Uncertainty |  |  | C | 1, 1 |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | (S) RE | 1, 1 |
| Uncertainty Variable Factor |  |  | (S) RE | 0, 1 |
| Vertical Uncertainty |  |  | C | 0, 1 |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | (S) RE | 1, 1 |
| Uncertainty Variable Factor |  |  | (S) RE | 0, 1 |
| Information |  |  | C | 0, 1 |
| File Locator |  |  | (S) TE | 0, 1 |
| File Reference | (TXTDSC) (NTXTDS) |  | (S) TE | 0, 1 |
| Headline |  |  | (S) TE | 0, 1 |
| language |  |  | (S) TE | 1, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 0, 1 |
| Textual Description |  |  | C | 0, 1 |
| File Reference | (TXTDSC) (NTXTDS) |  | (S) TE | 1, 1 (ordered) |
| language |  |  | (S) TE | 0, 1 (ordered) |

2. Geo Feature Types

2.1. Aids to Navigation

Definition: A visual, acoustical, or radio device, external to a ship, designed to assist in determining a safe course or a vessel's position, or to warn of dangers and/or obstructions. Aids to navigation usually include buoys, beacons, fog signals, lights, radio beacons, leading marks, radio position fixing systems and GNSS which are chart-related and assist safe navigation.

CamelCase: AidsToNavigation

Alias:

Super type:

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| Interoperability Identifier |  |  | UN | 0, 1 |
| Information |  |  | C | 0, * |
| File Locator |  |  | (S) TE | 0, 1 |
| File Reference | (TXTDSC) (NTXTDS) |  | (S) TE | 0, 1 |
| Headline |  |  | (S) TE | 0, 1 |
| language |  |  | (S) TE | 1, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 0, 1 |
| Feature Name |  |  | C | 0, * |
| language |  |  | (S) TE | 1, 1 |
| Name | (OBJNAM) |  | (S) TE | 1, 1 |
| Scale Minimum | (SCAMIN) |  | (S) IN | 0, 1 |
| Source Date | (SORDAT) |  | date | 0, 1 |
| Source |  |  | (S) TE | 0, 1 |
| Pictorial Representation | (PICREP) |  | (S) TE | 0, 1 |
| Installation Date |  |  | date | 0, 1 |
| Fixed Date Range |  |  | C | 0, 1 |
| Date End | (DATEND) |  | (S) DA | 0, 1 |
| Date Start | (DATSTA) |  | (S) DA | 0, 1 |
| Periodic Date Range |  |  | C | 0, 1 |
| Date End | (DATEND) |  | (S) DA | 1, 1 |
| Date Start | (DATSTA) |  | (S) DA | 1, 1 |
| Seasonal Action Required |  |  | (S) TE | 0, * |

Information Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| association | Aton Status |  |  |  | AtonStatusInformation | Statuspart | 0, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Aton Aggregations |  |  |  | AtonAggregation | peerAtonAggregation | 0, * |
| Asso | Aton Associations |  |  |  | AtonAssociation | peerAtonAssociation | 0, * |

2.2. Navigation Line

Definition: A straight line extending towards an area of navigational interest and generally generated by two navigational aids or one navigational aid and a bearing.

CamelCase: NavigationLine

Alias: NAVLNE

Super type: aidsToNavigation

Feature use type: geographic

Primitive: curve

Remarks: The extent of the navigation line depends on the visibility of the navigational aid(s). The recommended track is that portion of a 'navigation line' that a ship should use for navigation.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Navigation Line | (CATNAV) | 1 : Clearing Line 2 : Transit Line 3 : Leading Line Bearing a Recommended Track | EN | 1, 1 |
| Orientation |  |  | C | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Range System |  |  |  | RecommendedTrack | navigableTrack | 0, * |

2.3. Recommended Track

Definition: A route which has been specially examined to ensure so far as possible that it is free of dangers and along which ships are advised to navigate.

CamelCase: RecommendedTrack

Alias: RECTRC

Super type: aidsToNavigation

Feature use type: geographic

Primitive: curve

Remarks: Recommended tracks include all channels recommended for hydrographic reasons to lead safely between shoal depths. The use of such tracks is generally left to the discretion of the mariner and will depend on the vessel's draught, the state of the tide, adequacy of navigational aids and so on (IHO Chart Specifications, M-4). The recommended track is that portion of a 'navigation line' that a ship should use for navigation. In the case of a two-way recommended track only one value of orientation is encoded; the other value can be deduced (that is, the value encoded + 180 degrees).

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Based On Fixed Marks | (CATTRK) |  | BO | 1, 1 |
| Depth Range Minimum Value | (DRVAL1) |  | (S) RE | 0, 1 |
| Maximal Permitted Draught | (lg_drt) |  | (S) RE | 0, 1 |
| Orientation |  |  | C | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |
| Quality of Vertical Measurement | (QUASOU) | 1 : Depth Known 2 : Depth or Least Depth Unknown 3 : Doubtful Sounding 4 : Unreliable Sounding 5 : No Bottom Found at Value Shown 6 : Least Depth Known 7 : Least Depth Unknown, Safe Clearance at Value Shown 8 : Value Reported (Not Surveyed) 9 : Value Reported (Not Confirmed) 10 : Maintained Depth 11 : Not Regularly Maintained | EN | 0, * |
| Vertical Uncertainty |  |  | C | 0, 1 |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | (S) RE | 1, 1 |
| Uncertainty Variable Factor |  |  | (S) RE | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Technique of Vertical Measurement | (TECSOU) | 1 : Found by Echo Sounder 2 : Found by Side Scan Sonar 3 : Found by Multi Beam 4 : Found by Diver 5 : Found by Lead Line 6 : Swept by Wire-Drag 7 : Found by Laser 8 : Swept by Vertical Acoustic System 9 : Found by Electromagnetic Sensor 10 : Photogrammetry 11 : Satellite Imagery 12 : Found by Levelling 13 : Swept by Side Scan Sonar 14 : Computer Generated 15 : Found by LIDAR 16 : Synthetic Aperture Radar 17 : Hyperspectral Imagery 18 : Mechanically Swept | EN | 0, * |
| Traffic Flow | (TRAFIC) | 1 : Inbound 2 : Outbound 3 : One-Way 4 : Two-Way | EN | 1, 1 |
| Vertical Datum | (VERDAT) (Datum Level) (Reference Plane) (Levelling Datum) (Datum for Sounding Reduction) (Datum for Heights) | 1 : Mean Low Water Springs 2 : Mean Lower Low Water Springs 3 : Mean Sea Level 4 : Lowest Low Water 5 : Mean Low Water 6 : Lowest Low Water Springs 7 : Approximate Mean Low Water Springs 8 : Indian Spring Low Water 9 : Low Water Springs 10 : Approximate Lowest Astronomical Tide 11 : Nearly Lowest Low Water 12 : Mean Lower Low Water 13 : Low Water 14 : Approximate Mean Low Water 15 : Approximate Mean Lower Low Water 16 : Mean High Water 17 : Mean High Water Springs 18 : High Water 19 : Approximate Mean Sea Level 20 : High Water Springs 21 : Mean Higher High Water 22 : Equinoctial Spring Low Water 23 : Lowest Astronomical Tide 24 : Local Datum 25 : International Great Lakes Datum 1985 26 : Mean Water Level 27 : Lower Low Water Large Tide 28 : Higher High Water Large Tide 29 : Nearly Highest High Water 30 : Highest Astronomical Tide 31 : Local Low Water Reference Level 32 : Local High Water Reference Level 33 : Local Mean Water Reference Level 34 : Equivalent Height of Water (German GlW) 35 : Highest Shipping Height of Water (German HSW) 36 : Reference Low Water Level According to Danube Commission 37 : Highest Shipping Height of Water According to Danube Commission 38 : Dutch River Low Water Reference Level (OLR) 39 : Russian Project Water Level 40 : Russian Normal Backwater Level 41 : Ohio River Datum 43 : Dutch High Water Reference Level 44 : Baltic Sea Chart Datum 2000 45 : Dutch Estuary Low Water Reference Level (OLW) 46 : International Great Lakes Datum 2020 47 : Sea Floor 48 : Sea Surface 49 : Hydrographic Zero | EN | 0, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Range System |  |  |  | navigationLine | navigationLine | 1, * |

2.4. Equipment

Definition: The implements used in an operation or activity.

CamelCase: Equipment

Alias:

Super type: aidsToNavigation

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Structure Equipment |  |  |  | structureObject | parent | 1, 1 |

2.5. Structure Object

Definition: Something (such as a house, tower, bridge, etc.) that is built by putting parts together and that usually stands on its own.

CamelCase: StructureObject

Alias:

Super type: aidsToNavigation

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| AtoN Number |  |  | (S) TE | 1, 1 |
| Contact Address |  |  | C | 0, 1 |
| Delivery Point |  |  | (S) TE | 0, 1 |
| City Name |  |  | (S) TE | 0, 1 |
| Administrative Division |  |  | (S) TE | 0, 1 |
| Country Name |  |  | (S) TE | 0, 1 |
| Postal Code |  |  | (S) TE | 0, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Structure Equipment |  |  |  | equipment | child | 0, * |

2.6. Generic Buoy

Definition: A floating object moored to the bottom in a particular (charted) place, as an aid to navigation or for other specific purposes.

CamelCase: GenericBuoy

Alias:

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: Navigational buoys may be classified according to:  (a) their shape, appearance, or construction, such as barrel, can, cask, conical, cylindrical, dan, keg, nun, pillar, spar, spherical,or topmark buoy;  (b) their colour, such as black, chequered, green, red buoy;  (c) their location, such as bifurcation, fairway, junction, mid-channel, middle-ground, or turning buoy;  (d) the various kinds of hazards or dangers to navigation which they mark, such as bar, isolated danger, fish trap, obstruction, spoil ground, telegraph or wreck buoy;  (e) their particular purpose or use, such as anchor, anchorage, compass adjustment, dredging, farewell (or landfall), marker, quarantine, station (or watch), or warping buoy.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Buoy Shape | (BOYSHP) | 1 : Conical 2 : Can 3 : Spherical 4 : Pillar 5 : Spar 6 : Barrel 7 : Superbuoy 8 : Ice Buoy | EN | 1, 1 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Type of Buoy |  |  | (S) TE | 0, 1 |

2.7. Generic Beacon

Definition: A fixed artificial navigation mark that can be recognized by its shape, colour, pattern, topmark or light character, or a combination of these. It may carry various additional aids to navigation.

CamelCase: GenericBeacon

Alias:

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: This term is not commonly used when the navigation mark can be classified as a lighthouse.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Beacon Shape | (BCNSHP) | 1 : Stake, Pole, Perch, Post 2 : Withy 3 : Beacon Tower 4 : Lattice Beacon 5 : Pile Beacon 6 : Cairn 7 : Buoyant Beacon | EN | 1, 1 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Vertical Length | (VERLEN) |  | (S) RE | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Vertical Accuracy |  |  | (S) RE | 0, 1 |

2.8. Landmark

Definition: Any prominent object at a fixed location on land which can be used in determining a location or a direction.

CamelCase: Landmark

Alias: LNDMRK

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Landmark | (CATLMK) | 1 : Cairn 2 : Cemetery 3 : Chimney 4 : Dish Aerial 5 : Flagstaff 6 : Flare Stack 7 : Mast 8 : Windsock 9 : Monument 10 : Column/Pillar 11 : Memorial Plaque 12 : Obelisk 13 : Statue 14 : Cross 15 : Dome 16 : Radar Scanner 17 : Tower 18 : Windmill 19 : Windmotor 20 : Spire/Minaret 21 : Large Rock or Boulder on Land 22 : Triangulation Mark 23 : Boundary Mark 24 : Observation Wheel 25 : Torii 26 : Bridge 27 : Dam | EN | 1, * |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 1, 1 |
| Function | (FUNCTN) | 2 : Harbour-Masters Office 3 : Customs Office 4 : Health Office 5 : Hospital 6 : Post Office 7 : Hotel 8 : Railway Station 9 : Police Station 10 : Water-Police Station 11 : Pilot Office 12 : Pilot Lookout 13 : Bank Office 14 : Headquarters for District Control 15 : Transit Shed/Warehouse 16 : Factory 17 : Power Station 18 : Administrative 19 : Educational Facility 20 : Church 21 : Chapel 22 : Temple 23 : Pagoda 24 : Shinto Shrine 25 : Buddhist Temple 26 : Mosque 27 : Marabout 28 : Lookout 29 : Communication 30 : Television 31 : Radio 32 : Radar 33 : Light Support 34 : Microwave 35 : Cooling 36 : Observation 37 : Timeball 38 : Clock 39 : Control 40 : Airship Mooring 41 : Stadium 42 : Bus Station 43 : Passenger Terminal Building 44 : Sea Rescue Control 45 : Observatory 46 : Ore Crusher 47 : Boathouse 48 : Pumping Station 49 : Roof Above Navigable Water 50 : Building Above Navigable Water | EN | 0, * |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.9. Daymark

Definition: (1) The identifying characteristics of an aid to navigation which serve to facilitate its recognition against a daylight viewing background. On those structures that do not by themselves present an adequate viewing area to be seen at the required distance, the aid is made more visible by affixing a daymark to the structure. A daymark so affixed has a distinctive colour and shape depending on the purpose of the aid. (2) An unlighted navigational mark.

CamelCase: Daymark

Alias: DAYMAR

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Special Purpose Mark | (CATSPM) | 1 : Firing Danger Area Mark 2 : Target Mark 3 : Marker Ship Mark 4 : Degaussing Range Mark 5 : Barge Mark 6 : Cable Mark 7 : Spoil Ground Mark 8 : Outfall Mark 9 : ODAS 10 : Recording Mark 11 : Seaplane Anchorage Mark 12 : Recreation Zone Mark 13 : Private Mark 14 : Mooring Mark 15 : LANBY 16 : Leading Mark 17 : Measured Distance Mark 18 : Notice Mark 19 : TSS Mark 20 : Anchoring Prohibited Mark 21 : Berthing Prohibited Mark 22 : Overtaking Prohibited Mark 23 : Two-Way Traffic Prohibited Mark 24 : Reduced Wake Mark 25 : Speed Limit Mark 26 : Stop Mark 27 : General Warning Mark 28 : Sound Ship's Siren Mark 29 : Restricted Vertical Clearance Mark 30 : Maximum Vessel's Draught Mark 31 : Restricted Horizontal Clearance Mark 32 : Strong Current Warning Mark 33 : Berthing Permitted Mark 34 : Overhead Power Cable Mark 35 : Channel Edge Gradient Mark 36 : Telephone Mark 37 : Ferry Crossing Mark 39 : Pipeline Mark 40 : Anchorage Mark 41 : Clearing Mark 42 : Control Mark 43 : Diving Mark 44 : Refuge Beacon 45 : Foul Ground Mark 46 : Yachting Mark 47 : Heliport Mark 48 : GNSS Mark 49 : Seaplane Landing Mark 50 : Entry Prohibited Mark 51 : Work in Progress Mark 52 : Mark With Unknown Purpose 53 : Wellhead Mark 54 : Channel Separation Mark 55 : Marine Farm Mark 56 : Artificial Reef Mark 57 : Ice Mark 58 : Nature Reserve Mark 59 : Fish Aggregating Device 60 : Wreck Mark 61 : Customs Mark 62 : Causeway Mark 63 : Wave Recorder 64 : Jetski Prohibited | EN | 0, 1 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Topmark/Daymark Shape | (TOPSHP) | 1 : Cone (Point Up) 2 : Cone (Point Down) 3 : Sphere 4 : 2 Spheres 5 : Cylinder 6 : Board 7 : X-Shaped 8 : Upright Cross 9 : Cube (Point Up) 10 : 2 Cones (Point to Point) 11 : 2 Cones (Base to Base) 12 : Rhombus 13 : 2 Cones (Points Upward) 14 : 2 Cones (Points Downward) 15 : Besom (Point Up) 16 : Besom (Point Down) 17 : Flag 18 : Sphere Over a Rhombus 19 : Square 20 : Rectangle (Horizontal) 21 : Rectangle (Vertical) 22 : Trapezium (Up) 23 : Trapezium (Down) 24 : Triangle (Point Up) 25 : Triangle (Point Down) 26 : Circle 27 : Two Upright Crosses (One Over the Other) 28 : T-Shape 29 : Triangle Pointing Up Over a Circle 30 : Upright Cross Over a Circle 31 : Rhombus Over a Circle 32 : Circle Over a Triangle Pointing Up 33 : Other Shape (See Shape Information) 34 : Tubular | EN | 1, 1 |
| IsSlatted |  |  | BO | 1, 1 |

2.10. Fog Signal

Definition: A warning signal transmitted by a vessel, or aid to navigation, during periods of low visibility. Also, the device producing such a signal.

CamelCase: FogSignal

Alias: FOGSIG

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Fog Signal | (CATFOG) | 1 : Explosive 2 : Diaphone 3 : Siren 4 : Nautophone 5 : Reed 6 : Tyfon 7 : Bell 8 : Whistle 9 : Gong 10 : Horn | EN | 1, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Signal Sequence |  |  | C | 0, 1 |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |

2.11. Radar Reflector

Definition: A device capable of, or intended for, reflecting radar signals.

CamelCase: RadarReflector

Alias: RADRFL

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: A radar reflector is usually a tetrahedron or pentagonal corner reflector to facilitate reflection towards the sender (International Maritime Dictionary, 2nd Ed.).

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.12. Radar Transponder Beacon

Definition: A transponder beacon transmitting a coded signal on radar frequency, permitting an interrogating craft to determine the bearing and range of the transponder.

CamelCase: RadarTransponderBeacon

Alias: RTPBCN Radar Beacon RACON

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: The feature 'radar transponder beacon' is only used to encode the technical equipment independent of the structure on which it is located (for example a beacon, light-vessel or tower).

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Radar Transponder Beacon | (CATRTB) | 1 : Ramark, Radar Beacon Transmitting Continuously 2 : Racon, Radar Transponder Beacon 3 : Leading Racon/Radar Transponder Beacon | EN | 1, 1 |
| Sector Limit One |  |  | C | 0, 1 |
| Sector Bearing | (SECTR1) (SECTR2) |  | (S) RE | 1, 1 |
| Sector Line Length |  |  | (S) IN | 0, 1 |
| Sector Limit Two |  |  | C | 0, 1 |
| Sector Bearing | (SECTR1) (SECTR2) |  | (S) RE | 1, 1 |
| Sector Line Length |  |  | (S) IN | 0, 1 |
| Signal Group | (SIGGRP) |  | (S) TE | 0, 1 |
| Signal Sequence |  |  | C | 0, 1 |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Value of Nominal Range | (VALNMR) |  | (S) RE | 0, 1 |
| Radar Wave Length |  |  | C | 0, 1 |
| Radar Band |  |  | (S) TE | 1, 1 |
| Wave Length Value | (Radar Wave Length) |  | (S) RE | 1, 1 |

2.13. Radio Station

Definition: A place equipped to transmit radio waves. Such a station may be either stationary or mobile, and may also be provided with a radio receiver.

CamelCase: RadioStation

Alias: RDOSTA W/T Station

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: The transmission of a radio station may serve to provide mariners with a line of position (IHO Chart Specifications, M-4). The feature 'radio station' is used to encode the point of transmission of the signal.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Radio Station | (CATROS) | 20 : AIS Base Station | EN | 1, 1 |
| Estimated Range of Transmission | (ESTRNG) |  | (S) RE | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Physical AIS |  |  |  | physicalAISAidToNavigation | physicalAISbroadcastBy | 0, 1 |
| Asso | Synthetic AIS |  |  |  | SyntheticAISAidToNavigation | syntheticAISbroadcastBy | 0, 1 |
| Asso | Virtual AIS |  |  |  | VirtualAISAidToNavigation | virtualAISbroadcastBy | 0, 1 |

2.14. Retroreflector

Definition: A means of distinguishing unlighted marks at night. Retroreflective material is secured to the mark in a particular pattern to reflect back light.

CamelCase: Retroreflector

Alias: RETRFL

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: The body carrying the retro-reflector is a separate feature.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.15. Light Air Obstruction

Definition: An air obstruction light is a light marking an obstacle which constitutes a danger to air navigation.

CamelCase: LightAirObstruction

Alias: LIGHTS

Super type: GenericLight

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * |
| Light Visibility | (LITVIS) | 1 : High Intensity 2 : Low Intensity 3 : Faint 4 : Intensified 5 : Unintensified 6 : Visibility Deliberately Restricted 7 : Obscured 8 : Partially Obscured 9 : Visible in Line of Range | EN | 0, * |
| Exhibition Condition of Light | (EXCLIT) | 1 : Light Shown Without Change of Character 2 : Daytime Light 3 : Fog Light 4 : Night Light | EN | 0, 1 |
| Value of Nominal Range | (VALNMR) |  | (S) RE | 0, 1 |
| Flare Bearing |  |  | (S) IN | 0, 1 |
| Multiplicity of Features |  |  | C | 0, 1 |
| Multiplicity Known |  |  | BO | 1, 1 |
| Number of Features |  |  | (S) IN | 0, 1 |
| Rhythm Of Light |  |  | C | 1, 1 |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing 1 : group-occulting light 2 : composite group-occulting light 3 : group flashing light 4 : composite group-flashing light 5 : group quick light 6 : group very quick light | EN | 1, 1 |
| Signal Group | (SIGGRP) |  | (S) TE | 0, * |
| Signal Period |  |  | (S) RE | 0, 1 |
| Signal Sequence |  |  | C | 0, * |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |

2.16. Light All Around

Definition: An all around light is a light that is visible over the whole horizon of interest to marine navigation and having no change in the characteristics of the light.

CamelCase: LightAllAround

Alias: LIGHTS

Super type: GenericLight

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 3 : Red 4 : Green 5 : Blue 6 : Yellow 9 : Amber 10 : Violet 11 : Orange | EN | 1, * (ordered) |
| Signal Generation | (SIGGEN) | 1 : Automatically 2 : By Wave Action 3 : By Hand 4 : By Wind 5 : Radio Activated 6 : Call Activated | EN | 0, 1 |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 0, 1 |
| Major Light |  |  | BO | 0, 1 |
| Light Visibility | (LITVIS) | 1 : High Intensity 2 : Low Intensity 3 : Faint 4 : Intensified 5 : Unintensified 6 : Visibility Deliberately Restricted 7 : Obscured 8 : Partially Obscured 9 : Visible in Line of Range | EN | 0, 1 |
| Exhibition Condition of Light | (EXCLIT) | 1 : Light Shown Without Change of Character 2 : Daytime Light 3 : Fog Light 4 : Night Light | EN | 0, 1 |
| Category of Light | (CATLIT) | 4 : Leading Light 5 : Aero Light 8 : Flood Light 9 : Strip Light 10 : Subsidiary Light 11 : Spotlight 12 : Front 13 : Rear 14 : Lower 15 : Upper 17 : Emergency 18 : Bearing Light 19 : Horizontally Disposed 20 : Vertically Disposed | EN | 0, * |
| Value of Nominal Range | (VALNMR) |  | (S) RE | 0, 1 |
| Flare Bearing |  |  | (S) IN | 0, 1 |
| Multiplicity of Features |  |  | C | 0, 1 |
| Multiplicity Known |  |  | BO | 1, 1 |
| Number of Features |  |  | (S) IN | 0, 1 |
| Rhythm Of Light |  |  | C | 1, 1 |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing 1 : group-occulting light 2 : composite group-occulting light 3 : group flashing light 4 : composite group-flashing light 5 : group quick light 6 : group very quick light | EN | 1, 1 |
| Signal Group | (SIGGRP) |  | (S) TE | 0, * |
| Signal Period |  |  | (S) RE | 0, 1 |
| Signal Sequence |  |  | C | 0, * |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |

2.17. Light Fog Detector

Definition: A fog detector light is a light used to automatically determine conditions of visibility which warrant the turning on or off of a sound signal.

CamelCase: LightFogDetector

Alias: LIGHTS Fog Detector Light

Super type: GenericLight

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 3 : Red 4 : Green 5 : Blue 6 : Yellow 9 : Amber 10 : Violet 11 : Orange | EN | 0, * |
| Signal Generation | (SIGGEN) | 1 : Automatically 2 : By Wave Action 3 : By Hand 4 : By Wind 5 : Radio Activated 6 : Call Activated | EN | 0, 1 |
| Rhythm Of Light |  |  | C | 1, 1 |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing 1 : group-occulting light 2 : composite group-occulting light 3 : group flashing light 4 : composite group-flashing light 5 : group quick light 6 : group very quick light | EN | 1, 1 |
| Signal Group | (SIGGRP) |  | (S) TE | 0, * |
| Signal Period |  |  | (S) RE | 0, 1 |
| Signal Sequence |  |  | C | 0, * |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |

2.18. Light Sectored

Definition: A light presenting different appearances (in particular, different colours) over various parts of the horizon of interest to maritime navigation.

CamelCase: LightSectored

Alias: LIGHTS Sector Light

Super type: GenericLight

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Signal Generation | (SIGGEN) | 1 : Automatically 2 : By Wave Action 3 : By Hand 4 : By Wind 5 : Radio Activated 6 : Call Activated | EN | 0, 1 |
| Marks Navigational - System Of | (MARSYS) | 1 : IALA A 2 : IALA B 9 : No System 10 : Other System 11 : Main European Inland Waterway Marking System 12 : Russian Inland Waterway Regulations 13 : Brazilian National Inland Waterway Regulation 15 : Paraguay-Parana Waterway - Brazilian Complementary Aids | EN | 0, 1 |
| Exhibition Condition of Light | (EXCLIT) | 1 : Light Shown Without Change of Character 2 : Daytime Light 3 : Fog Light 4 : Night Light | EN | 0, 1 |
| Category of Light | (CATLIT) | 4 : Leading Light 5 : Aero Light 8 : Flood Light 9 : Strip Light 10 : Subsidiary Light 11 : Spotlight 12 : Front 13 : Rear 14 : Lower 15 : Upper 17 : Emergency 18 : Bearing Light 19 : Horizontally Disposed 20 : Vertically Disposed | EN | 0, * |
| Sector Characteristics |  |  | C | 1, * |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing 1 : group-occulting light 2 : composite group-occulting light 3 : group flashing light 4 : composite group-flashing light 5 : group quick light 6 : group very quick light | EN | 1, 1 |
| Light Sector |  |  | C | 1, 10 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * (ordered) |
| Directional Character |  |  | C | 0, 1 |
| Moire Effect |  |  | BO | 0, 1 |
| Orientation |  |  | C | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |
| Light Visibility | (LITVIS) | 1 : High Intensity 2 : Low Intensity 3 : Faint 4 : Intensified 5 : Unintensified 6 : Visibility Deliberately Restricted 7 : Obscured 8 : Partially Obscured 9 : Visible in Line of Range | EN | 0, * |
| Sector Limit |  |  | C | 0, 1 |
| Sector Limit One |  |  | C | 1, 1 |
| Sector Bearing | (SECTR1) (SECTR2) |  | (S) RE | 1, 1 |
| Sector Line Length |  |  | (S) IN | 0, 1 |
| Sector Limit Two |  |  | C | 1, 1 |
| Value of Nominal Range | (VALNMR) |  | (S) RE | 0, 1 |
| Sector Information |  |  | C | 0, * |
| language |  |  | (S) TE | 0, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 1, 1 |
| Sector Arc Extension |  |  | BO | 0, 1 |
| Signal Group | (SIGGRP) |  | (S) TE | 0, 10 (ordered) |
| Signal Period |  |  | (S) RE | 0, 1 |
| Signal Sequence |  |  | C | 0, 10 (ordered) |
| Signal Duration |  |  | (S) RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |
| Obscured Sector |  |  | C | 0, * |
| Sector Limit |  |  | C | 1, 1 |
| Sector Limit One |  |  | C | 1, 1 |
| Sector Bearing | (SECTR1) (SECTR2) |  | (S) RE | 1, 1 |
| Sector Line Length |  |  | (S) IN | 0, 1 |
| Sector Limit Two |  |  | C | 1, 1 |
| Sector Information |  |  | C | 0, 1 |
| language |  |  | (S) TE | 0, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 1, 1 |
| Multiplicity of Features |  |  | C | 0, 1 |
| Multiplicity Known |  |  | BO | 1, 1 |
| Number of Features |  |  | (S) IN | 0, 1 |

2.19. Light Float

Definition: A boat-like structure used instead of a light buoy in waters where strong streams or currents are experienced, or when a greater elevation than that of a light buoy is necessary.

CamelCase: LightFloat

Alias: LITFLT

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: The light of a light float is a separate feature, handled as with buoys, beacons, etc.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.20. Light Vessel

Definition: A distinctively marked vessel anchored or moored at a charted point, to serve as an aid to navigation. By night, it displays a characteristic light(s) and is usually equipped with other devices, such as fog signal, submarine sound signal, and radio-beacon, to assist navigation.

CamelCase: LightVessel

Alias: LITVES Lightship

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: The light(s), fog signal etc. of a light vessel are separate features, handled as with buoys, beacons, etc.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.21. Offshore Platform

Definition: A permanent offshore structure, either fixed or floating.

CamelCase: OffshorePlatform

Alias: OFSPLF

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Offshore Platform | (CATOFP) | 1 : Oil Rig 2 : Production Platform 3 : Observation/Research Platform 4 : Articulated Loading Platform 5 : Single Anchor Leg Mooring 6 : Mooring Tower 7 : Artificial Island 8 : Floating Production, Storage and Off-Loading Vessel 9 : Accommodation Platform 10 : Navigation, Communication and Control Buoy 11 : Floating Oil Tank | EN | 0, * |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Condition | (CONDTN) | 1 : Under Construction 2 : Ruined 3 : Under Reclamation 4 : Wingless 5 : Planned Construction | EN | 0, 1 |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.22. Pile

Definition: A long heavy timber or section of steel, wood, concrete, etc., forced into the earth or seafloor to serve as a support, as for a pier, or to resist lateral pressure; or as a free standing pole within a marine environment.

CamelCase: Pile

Alias: PILPNT

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Pile | (CATPLE) | 1 : Stake 3 : Post 4 : Tripodal 5 : Piling 6 : Area of Piles 7 : Pipe 8 : Mooring Post | EN | 0, 1 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |

2.23. Silo/Tank

Definition: A large storage structure used for storing loose materials, liquids and/or gases.

CamelCase: SiloTank

Alias: SILTNK

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Building Shape | (BUISHP) | 5 : High-Rise Building 6 : Pyramid 7 : Cylindrical 8 : Spherical 9 : Cubic | EN | 0, 1 |
| Category of Silo/Tank | (CATSIL) | 1 : Silo in General 2 : Tank in General 3 : Grain Elevator 4 : Water Tower | EN | 0, 1 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, 1 |
| Visual Prominence | (CONVIS) | 1 : Visually Conspicuous 2 : Not Visually Conspicuous 3 : Prominent | EN | 0, 1 |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.24. Cardinal Buoy

Definition: A cardinal buoy is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked.

CamelCase: CardinalBuoy

Alias: BOYCAR

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary S-32 5th Edition, 565). Topmark, light, fog signal, radar reflector and retro-reflector are separate objects.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Cardinal Mark | (CATCAM) | 1 : North Cardinal Mark 2 : East Cardinal Mark 3 : South Cardinal Mark 4 : West Cardinal Mark | EN | 1, 1 |

2.25. Emergency Wreck Marking Buoy

Definition: An emergency wreck marking buoy is a buoy moored on or above a new wreck, designed to provide a prominent (both visual and radio) and easily identifiable temporary first response.

CamelCase: EmergencyWreckMarkingBuoy

Alias:

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: No remarks.

2.26. Installation Buoy

Definition: An installation buoy is a buoy used for loading tankers with gas or oil.

CamelCase: InstallationBuoy

Alias: BOYINB

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565).

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category Of Installation Buoy |  | 1 : catenary anchor leg mooring (CALM) 2 : catenary anchor leg mooring (CALM) | EN | 1, 1 |

2.27. Isolated Danger Buoy

Definition: An isolated danger buoy is a buoy moored on or above an isolated danger of limited extent, which has navigable water all around it.

CamelCase: IsolatedDangerBuoy

Alias: BOYISD

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565). Topmark, light, fog signal, radar reflector and retro-reflector are separate objects.

2.28. Lateral Buoy

Definition: A lateral buoy is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well-defined channels and are used in conjunction with a conventional direction of buoyage.

CamelCase: LateralBuoy

Alias: BOYLAT

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565). Light, fog signal, radar reflector and retro-reflector are separate objects.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Lateral Mark | (CATLAM) | 1 : Port-Hand Lateral Mark 2 : Starboard-Hand Lateral Mark 3 : Preferred Channel to Starboard Lateral Mark 4 : Preferred Channel to Port Lateral Mark 5 : Right-Hand Side of the Waterway 6 : Left-Hand Side of the Waterway 7 : Right-Hand Side of the Channel 8 : Left-Hand Side of the Channel 9 : Bifurcation of the Waterway 10 : Bifurcation of the Channel 11 : Channel Near the Right Bank 12 : Channel Near the Left Bank 13 : Channel Cross-Over to the Right Bank 14 : Channel Cross-Over to the Left Bank 15 : Danger Point or Obstacles at the Right-Hand Side 16 : Danger Point or Obstacles at the Left-Hand Side 17 : Turn Off at the Right-Hand Side 18 : Turn Off at the Left-Hand Side 19 : Junction at the Right-Hand Side 20 : Junction at the Left-Hand Side 21 : Harbour Entry at the Right-Hand Side 22 : Harbour Entry at the Left-Hand Side 23 : Bridge Pier Mark 24 : Entry From a Lake to a Narrower Waterway, Right Bank 25 : Entry From a Lake to a Narrower Waterway, Left Bank 26 : Change Bank 27 : Continue Along Bank | EN | 1, 1 |

2.29. Mooring Buoy

Definition: A buoy secured to the bottom by permanent moorings with means for mooring a vessel by use of its anchor chain or mooring lines.

CamelCase: MooringBuoy

Alias:

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: No remarks.

2.30. Safe Water Buoy

Definition: A safe water buoy is used to indicate that there is navigable water around the mark.

CamelCase: SafeWaterBuoy

Alias: BOYSAW

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565). Topmark, light, fog signal, radar reflector and retro-reflector are separate objects.

2.31. Special Purpose/General Buoy

Definition: A special purpose buoy is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners.

CamelCase: SpecialPurposeGeneralBuoy

Alias: BOYSPP

Super type: genericBuoy

Feature use type: geographic

Primitive: point

Remarks: A buoy is a floating object moored to the bottom in a particular place, as an aid to navigation or for other specific purposes. (IHO Dictionary, S-32, 5th Edition, 565). Light, fog signal, radar reflector and retro-reflector are separate objects.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Special Purpose Mark | (CATSPM) | 1 : Firing Danger Area Mark 2 : Target Mark 3 : Marker Ship Mark 4 : Degaussing Range Mark 5 : Barge Mark 6 : Cable Mark 7 : Spoil Ground Mark 8 : Outfall Mark 9 : ODAS 10 : Recording Mark 11 : Seaplane Anchorage Mark 12 : Recreation Zone Mark 13 : Private Mark 14 : Mooring Mark 15 : LANBY 16 : Leading Mark 17 : Measured Distance Mark 18 : Notice Mark 19 : TSS Mark 20 : Anchoring Prohibited Mark 21 : Berthing Prohibited Mark 22 : Overtaking Prohibited Mark 23 : Two-Way Traffic Prohibited Mark 24 : Reduced Wake Mark 25 : Speed Limit Mark 26 : Stop Mark 27 : General Warning Mark 28 : Sound Ship's Siren Mark 29 : Restricted Vertical Clearance Mark 30 : Maximum Vessel's Draught Mark 31 : Restricted Horizontal Clearance Mark 32 : Strong Current Warning Mark 33 : Berthing Permitted Mark 34 : Overhead Power Cable Mark 35 : Channel Edge Gradient Mark 36 : Telephone Mark 37 : Ferry Crossing Mark 39 : Pipeline Mark 40 : Anchorage Mark 41 : Clearing Mark 42 : Control Mark 43 : Diving Mark 44 : Refuge Beacon 45 : Foul Ground Mark 46 : Yachting Mark 47 : Heliport Mark 48 : GNSS Mark 49 : Seaplane Landing Mark 50 : Entry Prohibited Mark 51 : Work in Progress Mark 52 : Mark With Unknown Purpose 53 : Wellhead Mark 54 : Channel Separation Mark 55 : Marine Farm Mark 56 : Artificial Reef Mark 57 : Ice Mark 58 : Nature Reserve Mark 59 : Fish Aggregating Device 60 : Wreck Mark 61 : Customs Mark 62 : Causeway Mark 63 : Wave Recorder 64 : Jetski Prohibited | EN | 1, * |

2.32. Cardinal Beacon

Definition: A cardinal beacon is used in conjunction with the compass to indicate where the mariner may find the best navigable water. It is placed in one of the four quadrants (North, East, South and West), bounded by inter-cardinal bearings from the point marked.

CamelCase: CardinalBeacon

Alias: BCNCAR

Super type: genericBeacon

Feature use type: geographic

Primitive: point

Remarks: A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). Light, fog signal, radar reflector and retro-reflector are separate objects.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Cardinal Mark | (CATCAM) | 1 : North Cardinal Mark 2 : East Cardinal Mark 3 : South Cardinal Mark 4 : West Cardinal Mark | EN | 1, 1 |

2.33. Isolated Danger Beacon

Definition: An isolated danger beacon is a beacon erected on an isolated danger of limited extent, which has navigable water all around it.

CamelCase: IsolatedDangerBeacon

Alias: BCNISD

Super type: genericBeacon

Feature use type: geographic

Primitive: point

Remarks: A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). Light, fog signal, radar reflector and retro-reflector are separate objects.

2.34. Safe Water Beacon

Definition: A safe water beacon is used to indicate that there is navigable water around the mark.

CamelCase: SafeWaterBeacon

Alias: BCNSAW

Super type: genericBeacon

Feature use type: geographic

Primitive: point

Remarks: A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). Light, fog signal, radar reflector and retro-reflector are separate objects.

2.35. Special Purpose/General Beacon

Definition: A special purpose beacon is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notices to Mariners.

CamelCase: SpecialPurposeGeneralBeacon

Alias: BCNSPP

Super type: genericBeacon

Feature use type: geographic

Primitive: point

Remarks: A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). Light, fog signal, radar reflector and retro-reflector are separate features.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Special Purpose Mark | (CATSPM) | 1 : Firing Danger Area Mark 2 : Target Mark 3 : Marker Ship Mark 4 : Degaussing Range Mark 5 : Barge Mark 6 : Cable Mark 7 : Spoil Ground Mark 8 : Outfall Mark 9 : ODAS 10 : Recording Mark 11 : Seaplane Anchorage Mark 12 : Recreation Zone Mark 13 : Private Mark 14 : Mooring Mark 15 : LANBY 16 : Leading Mark 17 : Measured Distance Mark 18 : Notice Mark 19 : TSS Mark 20 : Anchoring Prohibited Mark 21 : Berthing Prohibited Mark 22 : Overtaking Prohibited Mark 23 : Two-Way Traffic Prohibited Mark 24 : Reduced Wake Mark 25 : Speed Limit Mark 26 : Stop Mark 27 : General Warning Mark 28 : Sound Ship's Siren Mark 29 : Restricted Vertical Clearance Mark 30 : Maximum Vessel's Draught Mark 31 : Restricted Horizontal Clearance Mark 32 : Strong Current Warning Mark 33 : Berthing Permitted Mark 34 : Overhead Power Cable Mark 35 : Channel Edge Gradient Mark 36 : Telephone Mark 37 : Ferry Crossing Mark 39 : Pipeline Mark 40 : Anchorage Mark 41 : Clearing Mark 42 : Control Mark 43 : Diving Mark 44 : Refuge Beacon 45 : Foul Ground Mark 46 : Yachting Mark 47 : Heliport Mark 48 : GNSS Mark 49 : Seaplane Landing Mark 50 : Entry Prohibited Mark 51 : Work in Progress Mark 52 : Mark With Unknown Purpose 53 : Wellhead Mark 54 : Channel Separation Mark 55 : Marine Farm Mark 56 : Artificial Reef Mark 57 : Ice Mark 58 : Nature Reserve Mark 59 : Fish Aggregating Device 60 : Wreck Mark 61 : Customs Mark 62 : Causeway Mark 63 : Wave Recorder 64 : Jetski Prohibited | EN | 1, * |

2.36. Lateral Beacon

Definition: A lateral beacon is used to indicate the port or starboard hand side of the route to be followed. They are generally used for well defined channels and are used in conjunction with a conventional direction of buoyage.

CamelCase: LateralBeacon

Alias: BCNLAT

Super type: genericBeacon

Feature use type: geographic

Primitive: point

Remarks: A beacon is a prominent specially constructed object forming a conspicuous mark as a fixed aid to navigation or for use in hydrographic survey (IHO Dictionary, S-32, 5th Edition, 420). Light, fog signal, radar reflector and retro-reflector are separate objects.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category of Landmark | (CATLMK) | 1 : Cairn 2 : Cemetery 3 : Chimney 4 : Dish Aerial 5 : Flagstaff 6 : Flare Stack 7 : Mast 8 : Windsock 9 : Monument 10 : Column/Pillar 11 : Memorial Plaque 12 : Obelisk 13 : Statue 14 : Cross 15 : Dome 16 : Radar Scanner 17 : Tower 18 : Windmill 19 : Windmotor 20 : Spire/Minaret 21 : Large Rock or Boulder on Land 22 : Triangulation Mark 23 : Boundary Mark 24 : Observation Wheel 25 : Torii 26 : Bridge 27 : Dam | EN | 1, 1 |

2.37. Wind Turbine

Definition: A tower and associated equipment that generates electrical power from wind. They can be sited offshore and may be either fixed or floating.

CamelCase: WindTurbine

Alias: LNDMRK (CATLMK = 19)

Super type: structureObject

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, 1 |
| Condition | (CONDTN) | 1 : Under Construction 2 : Ruined 3 : Under Reclamation 4 : Wingless 5 : Planned Construction | EN | 0, 1 |
| Elevation | (ELEVAT) |  | (S) RE | 0, 1 |
| Height | (HEIGHT) |  | (S) RE | 0, 1 |
| Nature of Construction | (NATCON) | 6 : Wooden 7 : Metal 8 : Glass Reinforced Plastic 9 : Painted 10 : Framework 11 : Latticed 12 : Glass 13 : Fiberglass 14 : Plastic | EN | 0, * |
| Radar Conspicuous | (CONRAD) |  | BO | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.38. Generic Light

Definition: A source of light, usually artificial, that serves as an aid to navigation, providing visibility and guidance, especially in low-light conditions. It could be mounted on various structures like buoys, beacons, or lighthouses to indicate a specific location or navigational point.

CamelCase: GenericLight

Alias:

Super type:

Feature use type: geographic

Primitive: point

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Height | (HEIGHT) |  | RE | 0, 1 |
| Vertical Datum | (VERDAT) (Datum Level) (Reference Plane) (Levelling Datum) (Datum for Sounding Reduction) (Datum for Heights) | 1 : Mean Low Water Springs 2 : Mean Lower Low Water Springs 3 : Mean Sea Level 4 : Lowest Low Water 5 : Mean Low Water 6 : Lowest Low Water Springs 7 : Approximate Mean Low Water Springs 8 : Indian Spring Low Water 9 : Low Water Springs 10 : Approximate Lowest Astronomical Tide 11 : Nearly Lowest Low Water 12 : Mean Lower Low Water 13 : Low Water 14 : Approximate Mean Low Water 15 : Approximate Mean Lower Low Water 16 : Mean High Water 17 : Mean High Water Springs 18 : High Water 19 : Approximate Mean Sea Level 20 : High Water Springs 21 : Mean Higher High Water 22 : Equinoctial Spring Low Water 23 : Lowest Astronomical Tide 24 : Local Datum 25 : International Great Lakes Datum 1985 26 : Mean Water Level 27 : Lower Low Water Large Tide 28 : Higher High Water Large Tide 29 : Nearly Highest High Water 30 : Highest Astronomical Tide 31 : Local Low Water Reference Level 32 : Local High Water Reference Level 33 : Local Mean Water Reference Level 34 : Equivalent Height of Water (German GlW) 35 : Highest Shipping Height of Water (German HSW) 36 : Reference Low Water Level According to Danube Commission 37 : Highest Shipping Height of Water According to Danube Commission 38 : Dutch River Low Water Reference Level (OLR) 39 : Russian Project Water Level 40 : Russian Normal Backwater Level 41 : Ohio River Datum 43 : Dutch High Water Reference Level 44 : Baltic Sea Chart Datum 2000 45 : Dutch Estuary Low Water Reference Level (OLW) 46 : International Great Lakes Datum 2020 47 : Sea Floor 48 : Sea Surface 49 : Hydrographic Zero | EN | 0, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Effective Intensity |  |  | RE | 0, 1 |
| Peak Intensity |  |  | RE | 0, 1 |

2.39. Dangerous Feature

Definition: A characteristic or element in the environment that poses a potential risk to navigation or safety. This could include hazards such as rocks, submerged objects, shallow waters, or man-made structures that could endanger vessels or other forms of transportation.

CamelCase: DangerousFeature

Alias:

Super type:

Feature use type: geographic

Primitive: point

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Interoperability Identifier |  |  | UN | 1, 1 |
| Information |  |  | C | 0, * |
| File Locator |  |  | (S) TE | 0, 1 |
| File Reference | (TXTDSC) (NTXTDS) |  | (S) TE | 0, 1 |
| Headline |  |  | (S) TE | 0, 1 |
| language |  |  | (S) TE | 1, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 0, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Dangerous Feature Association | DangerousFeature | danger | 0, * | AtonAssociation | markingAton | 1, * |

2.40. Synthetic AIS Aid To Navigation

Definition: A virtual or synthetic Aid to Navigation (AtoN) transmitted via the Automatic Identification System (AIS). Unlike a physical AtoN, which exists in the real world (like buoys or lighthouses), a synthetic AIS AtoN is created in the AIS network to provide navigational information to mariners about a point of interest or hazard, even if no physical object is present at that location. It helps in improving situational awareness.

CamelCase: SyntheticAISAidToNavigation

Alias:

Super type: ElectronicAtoN

Feature use type: geographic

Primitive: point

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category Of Synthetic AIS Aid To Navigation |  | 1 : predicted 2 : monitored | EN | 1, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Synthetic AIS |  |  |  | radioStation | syntheticAISbroadcasts | 0, * |

2.41. Physical AIS Aid to Navigation

Definition: An Automatic Identification System (AIS) message 21 transmitted from a physical Aid to Navigation, or transmitted from an AIS station for an Aid to Navigation which physically exists.

CamelCase: PhysicalAISAidToNavigation

Alias:

Super type: ElectronicAtoN

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category Of Physical AIS Aid To Navigation |  | 1 : Physical AIS Type 1 2 : Physical AIS Type 2 3 : Physical AIS Type 3 |  | 1, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Physical AIS |  |  |  | radioStation | physicalAISbroadcasts | 0, * |

2.42. Virtual AIS Aid to Navigation

Definition: An Automatic Identification System (AIS) message 21 transmitted from an AIS station to simulate on navigation systems an Aid to Navigation which does not physically exist.

CamelCase: VirtualAISAidToNavigation

Alias:

Super type: ElectronicAtoN

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Virtual AIS Aid to Navigation Type |  | 1 : North Cardinal 2 : East Cardinal 3 : South Cardinal 4 : West Cardinal 5 : Port Lateral 6 : Starboard Lateral 7 : Preferred Channel to Port 8 : Preferred Channel to Starboard 9 : Isolated Danger 10 : Safe Water 11 : Special Purpose 12 : New Danger Marking | EN | 1, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Virtual AIS |  |  |  | radioStation | virtualAISbroadcasts | 0, * |

2.43. Electronic AtoN

Definition: -

CamelCase: ElectronicAtoN

Alias:

Super type: aidsToNavigation

Feature use type: geographic

Primitive: point

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| AtoN Number |  |  | TE | 1, 1 |
| MMSI Code |  |  | TE | 1, 1 |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |

2.44. Topmark

Definition: -

CamelCase: Topmark

Alias:

Super type: equipment

Feature use type: geographic

Primitive: point

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 0, * (ordered) |
| Colour Pattern | (COLPAT) | 1 : Horizontal Stripes 2 : Vertical Stripes 3 : Diagonal Stripes 4 : Squared 5 : Stripes (Direction Unknown) 6 : Border Stripe 7 : Single Colour 8 : Rectangle 9 : Triangle | EN | 0, * |
| Status | (STATUS) | 1 : Permanent 2 : Occasional 3 : Recommended 4 : Not in Use 5 : Periodic/Intermittent 6 : Reserved 7 : Temporary 8 : Private 9 : Mandatory 11 : Extinguished 12 : Illuminated 13 : Historic 14 : Public 15 : Synchronized 16 : Watched 17 : Unwatched 18 : Existence Doubtful 19 : On Request 20 : Drop Away 21 : Rising 22 : Increasing 23 : Decreasing 24 : Strong 25 : Good 26 : Moderately 27 : Poor 28 : Buoyed 29 : Fully Operational 30 : Partially Operational 31 : Drifting 32 : Broken 33 : Offline 34 : Discontinued 35 : Manual Observation 36 : Unknown Status 37 : Confirmed 38 : Candidate 39 : Under Modification 41 : Under Removal / Deletion 42 : Removed / Deleted 43 : Candidate for Modification | EN | 0, * |
| Topmark/Daymark Shape | (TOPSHP) | 1 : Cone (Point Up) 2 : Cone (Point Down) 3 : Sphere 4 : 2 Spheres 5 : Cylinder 6 : Board 7 : X-Shaped 8 : Upright Cross 9 : Cube (Point Up) 10 : 2 Cones (Point to Point) 11 : 2 Cones (Base to Base) 12 : Rhombus 13 : 2 Cones (Points Upward) 14 : 2 Cones (Points Downward) 15 : Besom (Point Up) 16 : Besom (Point Down) 17 : Flag 18 : Sphere Over a Rhombus 19 : Square 20 : Rectangle (Horizontal) 21 : Rectangle (Vertical) 22 : Trapezium (Up) 23 : Trapezium (Down) 24 : Triangle (Point Up) 25 : Triangle (Point Down) 26 : Circle 27 : Two Upright Crosses (One Over the Other) 28 : T-Shape 29 : Triangle Pointing Up Over a Circle 30 : Upright Cross Over a Circle 31 : Rhombus Over a Circle 32 : Circle Over a Triangle Pointing Up 33 : Other Shape (See Shape Information) 34 : Tubular | EN | 1, 1 |

2.45. Aton Aggregation

Definition: Used to identify an aggregation of two or more objects. This aggregation may be named content of categoryOfAggregation should be put in information attribute when converting to S-57.

CamelCase: AtonAggregation

Alias:

Super type:

Feature use type: geographic

Primitive: noGeometry

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category Of Aggregation |  | 1 : leading line 3 : measured distance 2 : range system |  | 1, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Aton Aggregations |  |  |  | aidsToNavigation | atonAggregationBy | 0, * |

2.46. Aton Association

Definition: Used to identify an association between two or more objects. The association may be named content of categoryOfAssociation should be put in information attribute when converting to S-57

CamelCase: AtonAssociation

Alias:

Super type:

Feature use type: geographic

Primitive: noGeometry

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Category Of Association |  | 1 : channel markings 2 : danger markings |  | 1, 1 |

Feature Bindings:

| Type | Association Name | Association Ends |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Class | Role | Mult | Class | Role | Mult |
| Asso | Dangerous Feature Association | AtonAssociation | markingAton | 1, * | DangerousFeature | danger | 0, * |
| Asso | Aton Associations |  |  |  | aidsToNavigation | atonAssociationBy | 0, * |

3. Carto Feature Types

4. Information Types

4.1. Spatial Quality

Definition: The indication of the quality of the locational information for features in a dataset.

CamelCase: SpatialQuality

Alias:

Super type:

Remarks: No remarks.

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Quality of Vertical Measurement | (QUASOU) | 1 : Depth Known 2 : Depth or Least Depth Unknown 3 : Doubtful Sounding 4 : Unreliable Sounding 5 : No Bottom Found at Value Shown 6 : Least Depth Known 7 : Least Depth Unknown, Safe Clearance at Value Shown 8 : Value Reported (Not Surveyed) 9 : Value Reported (Not Confirmed) 10 : Maintained Depth 11 : Not Regularly Maintained | EN | 0, 1 |
| Spatial Accuracy |  |  | C | 0, 1 |
| Fixed Date Range |  |  | C | 0, 1 |
| Date End | (DATEND) |  | (S) DA | 0, 1 |
| Date Start | (DATSTA) |  | (S) DA | 0, 1 |
| Horizontal Position Uncertainty |  |  | C | 0, 1 |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | (S) RE | 1, 1 |
| Uncertainty Variable Factor |  |  | (S) RE | 0, 1 |
| Vertical Uncertainty |  |  | C | 0, 1 |

4.2. Aton Status Information

Definition: This refers to the current operational status or condition of an Aid to Navigation (AtoN). It provides details about whether the navigational aid (such as a buoy, light, or beacon) is functioning properly, temporarily out of service, under maintenance, or has any other status that affects its operation or visibility to mariners.

CamelCase: AtonStatusInformation

Alias:

Super type:

Remarks: -

Attribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Change Details |  |  | C | 1, 1 |
| Aton Commissioning |  | 1 : Buoy establishment 2 : Light establishment 3 : Beacon establishment 4 : Audible signal establishment 5 : Fog signal establishment 6 : AIS transmitter establishment 7 : V-AIS establishment 8 : RACON establishment 9 : DGPS station establishment 10 : eLoran station establishment 11 : DGLONASS station establishment 12 : e-Chayka station establishment 13 : EGNOS station establishment | EN | 0, 1 |
| Aton Removal |  | 1 : Buoy removal 2 : Buoy temporary removal 3 : Light removal 4 : Light temporary removal 5 : Beacon removal 6 : Beacon temporary removal 7 : Fog signal removal 8 : Fog signal temporary removal 9 : Audible signal removal 10 : Audible signal temporary removal 11 : AIS transmitter removal 12 : AIS transmitter temporary removal 13 : V-AIS removal 14 : V-AIS temporary removal 15 : RACON removal 16 : RACON temporary removal 17 : DGPS station removal 18 : DGPS station temporary removal 19 : EGNOS station removal 20 : EGNOS station temporary removal 21 : LORAN station removal 22 : LORAN station temporary removal 24 : eLORAN station temporary removal 25 : Chayka station removal 26 : Chayka station temporary removal 27 : e-Chayka station removal | EN | 0, 1 |
| Aton Replacement |  | 1 : Buoy change 2 : Buoy temporary change 3 : Light change 4 : Light temporary change 5 : Sector light change 6 : Sector light temporary change 7 : Beacon change 8 : Beaon temporary change 9 : Fog signal change 10 : Fog signal temporary chagne 11 : Audible signal change 12 : Audible signal temporary change 13 : V-AIS change 14 : V-AIS temporary change 15 : RACON change 16 : RACON temporary change | EN | 0, 1 |
| Fixed Aton Change |  | 1 : Beacon missing 2 : Beacon damaged 3 : Light beacon Unlit 4 : Light beacon Unreliable 5 : Light beacon Not synchronized 6 : Light beacon damaged 7 : Beacon topmark missing 8 : Beacon topmark damaged 9 : Beacon daymark unreliable 10 : Floodlit beacon Unlit 11 : Beacon restored to normal | EN | 0, 1 |
| Floating Aton Change |  | 1 : Buoy adrift 2 : Buoy damaged 3 : Buoy daymark unreliable 4 : Buoy destroyed 5 : Buoy missing 6 : Buoy move 7 : Buoy off position 8 : Buoy re-establishment 9 : Buoy restored to normal 10 : Buoy topmark damaged 11 : Buoy topmark missing 12 : Buoy will be withdrawn 13 : Buoy withdrawn 14 : Decommissioned for winter 15 : Lifted for Winter 16 : Light buoy Light damaged 17 : Light buoy Light not sychronized 18 : Light buoy Light unlit 19 : Light buoy Light unreliable 20 : Marine Aids to Navigation unreliable 21 : Recommissioned for navigation season 22 : Replaced by Winter Spar 23 : Seasonal decommissioning complete 24 : Seasonal decommissioning in progress 25 : Seasonal recommissioning complete 26 : Seasonal recommissioning in progress | EN | 0, 1 |
| Audible Signal Aton Change |  | 1 : Audible signal out of service 2 : Fog signal out of service 3 : Audible signal operating properly 4 : Fog signal operating properly | EN | 0, 1 |
| Lighted Aton Change |  | 1 : Light unlit 2 : Light unreliable 3 : Light re-establishment 4 : Light range reduced 5 : Light without rhythm 6 : Light out of synchronization 7 : Light daymark unreliable 8 : Light operating properly 9 : Sector light Sector obscured 10 : Front leading/rnage light Unlit 11 : Rear leading/range light Unlit 12 : Front leading/range light Unreliable 13 : Rear leading/range light Unreliable 14 : Front leading/range light Light range reduced 15 : Rear leading/range light Light range reduced 16 : Front leading/range light without rhythm 17 : Rear leading/range light without rhythm 18 : Leading/range lights out of synchronization 19 : Front leading/range beacon Unreliable 20 : Rear leading/range beacon Unreliable 21 : Front leading/range light is operating properly 22 : Rear leading/range light is operating properly 23 : Front leading/range beacon restored to normal 24 : Rear leading/range beacon restored to normal | EN | 0, 1 |
| Electronic Aton Change |  | 1 : AIS transmitter out of service 2 : AIS transmitter unreliable 3 : AIS trnasmitter operating properly 4 : V-AIS out of service 5 : V-AIS unrealiable 6 : V-AIS operating properly 7 : RACON out of service 8 : RACON unreliable 9 : RACON operating properly 10 : DGPS out of service 11 : DGPS operating properly 12 : DGPS unreliable 13 : LORAN C operating properly 14 : LORAN C unreliable 15 : LORAN C out of service 16 : eLORAN operating properly 17 : eLORAN unreliable 18 : eLORAN out of service 19 : DGLONASS operating properly 20 : DGLONASS unreliable 21 : DGLONASS out of service 22 : Chayka operating properly 23 : Chayka unreliable 24 : Chayka out of service 25 : e-Chayka operating properly 26 : e-Chayka unreliable 27 : e-Chayka out of service 28 : EGNOS operating properly 29 : EGNOS unreliable 30 : EGNOS out of service | EN | 0, 1 |
| Change Types |  | 1 : Advanced notice of changes 2 : Discrepancy 3 : Proposed changes 4 : Temporary changes | EN | 0, 1 |

5. Simple Attributes

5.1. File Locator

Definition: The location of a fragment of text or other information in a support file.

CamelCase: fileLocator

Alias:

Value type: text

Remarks: Application schemas must describe how the associated file is identified. The associated file will commonly be named in a file reference co-attribute of the same complex attribute. Each DCEG must specify requirements for the format of the associated file and the semantics of file locator. For example, the value of file locator may be an HTML ID in an HTML file, line number in a text file) or a bookmark in a PDF file.

5.2. Uncertainty Fixed

Definition: The best estimate of the fixed horizontal or vertical accuracy component for positions, depths, heights, vertical distances and vertical clearances.

CamelCase: uncertaintyFixed

Alias: POSACC SOUACC VERACC

Value type: real

Remarks: No remarks.

5.3. Uncertainty Variable Factor

Definition: The factor to be applied to the variable component of an uncertainty equation so as to provide the best estimate of the variable horizontal or vertical accuracy component for positions, depths, heights, vertical distances and vertical clearances.

CamelCase: uncertaintyVariableFactor

Alias:

Value type: real

Remarks: No remarks.

5.4. Virtual AIS Aid to Navigation Type

Definition: A purpose of a virtual AIS Aid to Navigation.

CamelCase: virtualAISAidToNavigationType

Alias:

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | North Cardinal | Indicates that it should be passed to the north side of the aid. |
| 2 | East Cardinal | Indicates that it should be passed to the east side of the aid. |
| 3 | South Cardinal | Indicates that it should be passed to the south side of the aid. |
| 4 | West Cardinal | Indicates that it should be passed to the west side of the aid. |
| 5 | Port Lateral | Indicates the port boundary of a navigational channel or suggested route when proceeding in the conventional direction of buoyage. |
| 6 | Starboard Lateral | Indicates the starboard boundary of a navigational channel or suggested route when proceeding in the conventional direction of buoyage. |
| 7 | Preferred Channel to Port | At a point where a channel divides, when proceeding in the conventional direction of buoyage, the preferred channel (or primary route) is indicated by a modified port-hand lateral mark. |
| 8 | Preferred Channel to Starboard | At a point where a channel divides, when proceeding in the conventional direction of buoyage, the preferred channel (or primary route) is indicated by a modified starboard-hand lateral mark. |
| 9 | Isolated Danger | A mark used alone to indicate a dangerous reef or shoal. The mark may be passed on either hand. |
| 10 | Safe Water | Indicates that there is navigable water around the mark. |
| 11 | Special Purpose | A special purpose aid is primarily used to indicate an area or feature, the nature of which is apparent from reference to a chart, Sailing Directions or Notice to Mariners |
| 12 | New Danger Marking | A mark used to indicate the existence of a recently identified new danger, such as a wreck. |

5.5. Aton Commissioning

Definition: The process of deploying and activating a new Aid to Navigation (AtoN), ensuring that it is properly installed and operational.

CamelCase: atonCommissioning

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Buoy establishment | - |
| 2 | Light establishment | - |
| 3 | Beacon establishment | - |
| 4 | Audible signal establishment | - |
| 5 | Fog signal establishment | - |
| 6 | AIS transmitter establishment | - |
| 7 | V-AIS establishment | - |
| 8 | RACON establishment | - |
| 9 | DGPS station establishment | - |
| 10 | eLoran station establishment | - |
| 11 | DGLONASS station establishment | - |
| 12 | e-Chayka station establishment | - |
| 13 | EGNOS station establishment | - |

5.6. Aton Removal

Definition: The process of decommissioning and physically removing an AtoN from its designated location, either temporarily or permanently.

CamelCase: atonRemoval

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Buoy removal |  |
| 2 | Buoy temporary removal |  |
| 3 | Light removal |  |
| 4 | Light temporary removal |  |
| 5 | Beacon removal |  |
| 6 | Beacon temporary removal |  |
| 7 | Fog signal removal |  |
| 8 | Fog signal temporary removal |  |
| 9 | Audible signal removal |  |
| 10 | Audible signal temporary removal |  |
| 11 | AIS transmitter removal |  |
| 12 | AIS transmitter temporary removal |  |
| 13 | V-AIS removal |  |
| 14 | V-AIS temporary removal |  |
| 15 | RACON removal |  |
| 16 | RACON temporary removal |  |
| 17 | DGPS station removal |  |
| 18 | DGPS station temporary removal |  |
| 19 | EGNOS station removal |  |
| 20 | EGNOS station temporary removal |  |
| 21 | LORAN station removal |  |
| 22 | LORAN station temporary removal |  |
| 24 | eLORAN station temporary removal |  |
| 25 | Chayka station removal |  |
| 26 | Chayka station temporary removal |  |
| 27 | e-Chayka station removal |  |

5.7. Aton Replacement

Definition: The act of swapping an existing AtoN with a new or upgraded unit, either due to maintenance needs or technological improvements.

CamelCase: atonReplacement

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Buoy change | - |
| 2 | Buoy temporary change | - |
| 3 | Light change | - |
| 4 | Light temporary change | - |
| 5 | Sector light change | - |
| 6 | Sector light temporary change | - |
| 7 | Beacon change | - |
| 8 | Beaon temporary change | - |
| 9 | Fog signal change | - |
| 10 | Fog signal temporary chagne | - |
| 11 | Audible signal change | - |
| 12 | Audible signal temporary change | - |
| 13 | V-AIS change | - |
| 14 | V-AIS temporary change | - |
| 15 | RACON change | - |
| 16 | RACON temporary change | - |

5.8. Fixed Aton Change

Definition: Modifications or updates to fixed AtoNs, such as lighthouses or beacons, which are permanently positioned.

CamelCase: fixedAtonChange

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Beacon missing | - |
| 2 | Beacon damaged | - |
| 3 | Light beacon Unlit | - |
| 4 | Light beacon Unreliable | - |
| 5 | Light beacon Not synchronized | - |
| 6 | Light beacon damaged | - |
| 7 | Beacon topmark missing | - |
| 8 | Beacon topmark damaged | - |
| 9 | Beacon daymark unreliable | - |
| 10 | Floodlit beacon Unlit | - |
| 11 | Beacon restored to normal | - |

5.9. Moire Effect

Definition: A short range (up to 2km) type of directional light. Sodium lighting gives a yellow background to a screen on which a vertical black line will be seen by an observer on the centre line.

CamelCase: moireEffect

Alias:

Value type: boolean

Remarks: No remarks.

5.10. Orientation Value

Definition: The angular distance measured from true north to the major axis of the feature.

CamelCase: orientationValue

Alias: ORIENT

Value type: real

Remarks: No remarks.

5.11. Floating Aton Change

Definition: Adjustments or replacements related to floating AtoNs, such as buoys, which are anchored but can move with water currents.

CamelCase: floatingAtonChange

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Buoy adrift | - |
| 2 | Buoy damaged | - |
| 3 | Buoy daymark unreliable | - |
| 4 | Buoy destroyed | - |
| 5 | Buoy missing | - |
| 6 | Buoy move | - |
| 7 | Buoy off position | - |
| 8 | Buoy re-establishment | - |
| 9 | Buoy restored to normal | - |
| 10 | Buoy topmark damaged | - |
| 11 | Buoy topmark missing | - |
| 12 | Buoy will be withdrawn | - |
| 13 | Buoy withdrawn | - |
| 14 | Decommissioned for winter | - |
| 15 | Lifted for Winter | - |
| 16 | Light buoy Light damaged | - |
| 17 | Light buoy Light not sychronized | - |
| 18 | Light buoy Light unlit | - |
| 19 | Light buoy Light unreliable | - |
| 20 | Marine Aids to Navigation unreliable | - |
| 21 | Recommissioned for navigation season | - |
| 22 | Replaced by Winter Spar | - |
| 23 | Seasonal decommissioning complete | - |
| 24 | Seasonal decommissioning in progress | - |
| 25 | Seasonal recommissioning complete | - |
| 26 | Seasonal recommissioning in progress | - |

5.12. Seasonal Action Required

Definition: Indicates whether specific actions or maintenance must be performed based on the season (for example, repositioning, removal, or other adjustments).

CamelCase: seasonalActionRequired

Alias:

Value type: text

Remarks: -

5.13. Audible Signal Aton Change

Definition: Any modification to an AtoN that uses sound signals, such as foghorns or bells, to assist in navigation under low visibility conditions.

CamelCase: audibleSignalAtonChange

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Audible signal out of service | - |
| 2 | Fog signal out of service | - |
| 3 | Audible signal operating properly | - |
| 4 | Fog signal operating properly | - |

5.14. Signal Status

Definition: The indication of an element of a signal sequence being a period of light/sound or eclipse/silence.

CamelCase: signalStatus

Alias:

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Lit/Sound | The indication of an element of a signal sequence being a period of light or sound. |
| 2 | Eclipsed/Silent | The indication of an element of a signal sequence being a period of eclipse or silence. |

5.15. Date Start

Definition: The earliest date on which an object (for example a buoy) will be present.

CamelCase: dateStart

Alias: DATSTA

Value type: S100_TruncatedDate

Remarks: The Date Start should be encoded using 4 digits for the calendar year (YYYY), 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, indication of the month and/or day is omitted, and replaced with dashes (-). When no specific year is required (that is, the event or date range ends at the same time each year) the following two cases may be considered:- same day each year: ----MMDD- same month each year: ----MM--This conforms to ISO 8601: 2004. Date Start indicates the earliest date of an event or the start of a date range. It is used to indicate the start of a fixed date range, the start of a periodic date range, or the deployment or implementation of a feature at a specific date in the future.

5.16. Date End

Definition: The latest date on which an object (for example a buoy) will be present.

CamelCase: dateEnd

Alias: DATEND

Value type: S100_TruncatedDate

Remarks: The Date End should be encoded using 4 digits for the calendar year (YYYY), 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, indication of the month and/or day is omitted, and replaced with dashes (-). When no specific year is required (that is, the event or date range ends at the same time each year) the following two cases may be considered:- same day each year: ----MMDD- same month each year: ----MM--This conforms to ISO 8601: 2004. Date End indicates the latest date of an event or the end of a date range. It is used to indicate the end of a fixed date range, the end of a periodic date range, or the removal or cancellation of a feature at a specific date in the future.

5.17. Lighted Aton Change

Definition: Updates or modifications to light-emitting AtoNs, including changing light characteristics, intensity, or operational status.

CamelCase: lightedAtonChange

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Light unlit | - |
| 2 | Light unreliable | - |
| 3 | Light re-establishment | - |
| 4 | Light range reduced | - |
| 5 | Light without rhythm | - |
| 6 | Light out of synchronization | - |
| 7 | Light daymark unreliable | - |
| 8 | Light operating properly | - |
| 9 | Sector light Sector obscured | - |
| 10 | Front leading/rnage light Unlit | - |
| 11 | Rear leading/range light Unlit | - |
| 12 | Front leading/range light Unreliable | - |
| 13 | Rear leading/range light Unreliable | - |
| 14 | Front leading/range light Light range reduced | - |
| 15 | Rear leading/range light Light range reduced | - |
| 16 | Front leading/range light without rhythm | - |
| 17 | Rear leading/range light without rhythm | - |
| 18 | Leading/range lights out of synchronization | - |
| 19 | Front leading/range beacon Unreliable | - |
| 20 | Rear leading/range beacon Unreliable | - |
| 21 | Front leading/range light is operating properly | - |
| 22 | Rear leading/range light is operating properly | - |
| 23 | Front leading/range beacon restored to normal | - |
| 24 | Rear leading/range beacon restored to normal | - |

5.18. Text

Definition: A non-formatted digital text string.

CamelCase: text

Alias: INFORM NINFOM

Value type: text

Remarks: Should be used, for example, to hold the information that is for short cautionary or explanatory notes. Therefore, text populated in text must not exceed 300 characters. Text may be in English, or in a national language. No formatting of text is possible within text. If formatted text, or text strings exceeding 300 characters, is required, then an alternate concept should be used.

5.19. Headline

Definition: Words set at the head of a passage or page to introduce or categorize.

CamelCase: headline

Alias:

Value type: text

Remarks: No remarks.

5.20. Sector Line Length

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference. Sector line length specifies the displayed length of the line, in ground units, defining the limit of the sector.

CamelCase: sectorLineLength

Alias:

Value type: integer

Remarks: No remarks.

5.21. Signal Duration

Definition: The time occupied by a single instance of light/sound or eclipse/silence in a signal sequence.

CamelCase: signalDuration

Alias:

Value type: real

Remarks: No remarks.

5.22. Sector Bearing

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference. Sector bearing specifies the limit of the sector.

CamelCase: sectorBearing

Alias: SECTR1 SECTR2

Value type: real

Remarks: The values given to the common limits of adjacent sectors should be identical. The orientation of bearing is from seaward to the central object. This conforms with the method used in 'List of Lights' publications. A generic term such as 'to shore' cannot be used; a specific bearing must be encoded. Where a light sector limit is defined as 'to the shore', it should be encoded using a value that ensures that, when the limit is drawn, it will fall entirely on land.

5.23. Wave Length Value

Definition: The distance between two successive peaks (or other points of identical phase) on an electromagnetic wave.

CamelCase: waveLengthValue

Alias: Radar Wave Length

Value type: real

Remarks: No remarks.

5.24. Radar Band

Definition: The band code character of the electromagnetic spectrum within which radar wave lengths lie.

CamelCase: radarBand

Alias:

Value type: text

Remarks: No remarks.

5.25. Number of Features

Definition: The number of features of identical character that exist as a co-located group.

CamelCase: numberOfFeatures

Alias:

Value type: integer

Remarks: No remarks.

5.26. Multiplicity Known

Definition: The number of features of identical character that exist as a co-located group is or is not known.

CamelCase: multiplicityKnown

Alias:

Value type: boolean

Remarks: No remarks.

5.27. Sector Arc Extension

Definition: 	An indication that the default radius of a sector arc is to be extended by 5mm.

CamelCase: sectorArcExtension

Alias:

Value type: boolean

Remarks: No remarks.

5.28. File Reference

Definition: The file name of an externally referenced text file.

CamelCase: fileReference

Alias: TXTDSC NTXTDS

Value type: text

Remarks: No remarks.

5.29. Electronic Aton Change

Definition: Modifications to electronic or digital AtoNs, such as AIS (Automatic Identification System) AtoNs, virtual AtoNs, or remote-controlled systems.

CamelCase: electronicAtoNChange

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | AIS transmitter out of service | - |
| 2 | AIS transmitter unreliable | - |
| 3 | AIS trnasmitter operating properly | - |
| 4 | V-AIS out of service | - |
| 5 | V-AIS unrealiable | - |
| 6 | V-AIS operating properly | - |
| 7 | RACON out of service | - |
| 8 | RACON unreliable | - |
| 9 | RACON operating properly | - |
| 10 | DGPS out of service | - |
| 11 | DGPS operating properly | - |
| 12 | DGPS unreliable | - |
| 13 | LORAN C operating properly | - |
| 14 | LORAN C unreliable | - |
| 15 | LORAN C out of service | - |
| 16 | eLORAN operating properly | - |
| 17 | eLORAN unreliable | - |
| 18 | eLORAN out of service | - |
| 19 | DGLONASS operating properly | - |
| 20 | DGLONASS unreliable | - |
| 21 | DGLONASS out of service | - |
| 22 | Chayka operating properly | - |
| 23 | Chayka unreliable | - |
| 24 | Chayka out of service | - |
| 25 | e-Chayka operating properly | - |
| 26 | e-Chayka unreliable | - |
| 27 | e-Chayka out of service | - |
| 28 | EGNOS operating properly | - |
| 29 | EGNOS unreliable | - |
| 30 | EGNOS out of service | - |

5.30. Category Of Association

Definition: named associations between two or more aids to navigation and/or navigationally relevant features

CamelCase: categoryOfAssociation

Alias:

Value type: S100_CodeList

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | channel markings | - |
| 2 | danger markings | - |

5.31. Category Of Aggregation

Definition: named aggregations between two or more aids to navigation and/or navigationally relevant features

CamelCase: categoryOfAggregation

Alias:

Value type: S100_CodeList

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | leading line | - |
| 3 | measured distance | - |
| 2 | range system | - |

5.32. MMSI Code

Definition: The Maritime Mobile Service Identity (MMSI) Code is formed of a series of nine digits which are transmitted over the radio path in order to uniquely identify ship stations, ship earth stations,coast stations, coast earth stations, and group calls. These identities are formed in such a way that the identity or part thereof can be used by telephone and telex subscribers connected to the general telecommunications network principally to call ships automatically.

CamelCase: mMSICode

Alias:

Value type: text

Remarks: No remarks.

5.33. Category Of Installation Buoy

Definition: Classification of fixed installation buoy

CamelCase: categoryOfInstallationBuoy

Alias:

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | catenary anchor leg mooring (CALM) | incorporates a large buoy which remains on the surface at all times and is moored by 4 or more anchors. Mooring hawsers and cargo hoses lead from a turntable on top of the buoy, so that the buoy does not turn as the ship swings to wind and stream. |
| 2 | catenary anchor leg mooring (CALM) | a mooring structure used by tankers to load and unload in port approaches or in offshore oil and gas fields. The size of the structure can vary between a large mooring buoy and a manned floating structure. Also known as single point mooring (SPM) |

5.34. Light Characteristic

Definition: -

CamelCase: lightCharacteristic

Alias: LITCHR Character of Light

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Fixed | A signal light that shows continuously, in any given direction, with constant luminous intensity and colour. |
| 2 | Flashing | A rhythmic light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the appearances of light are of equal duration. |
| 3 | Long-Flashing | A single-flashing light in which an appearance of light of not less than two seconds duration is regularly repeated. |
| 4 | Quick-Flashing | A rhythmic light in which flashes are repeated at a rate of not less than 50 flashes per minutes but less than 80 flashes per minutes. It may be: - Continuous quick-flashing: A quick-flashing light in which a flash is regularly repeated. - Group quick-flashing: A quick-flashing light in which a group of two or more flashes, which are specified in number, is regularly repeated. |
| 5 | Very Quick-Flashing | A rhythmic light in which flashes are repeated at a rate of not less than 80 flashes per minute but less than 160 flashes per minute. It may be:- Continuous very quick-flashing: A very quick-flashing light in which a flash is regularly repeated.- Group very quick-flashing: A very quick-flashing light in which a group of two or more flashes, which are specified in number, is regularly repeated. |
| 6 | Continuous Ultra Quick-Flashing | A rhythmic light in which flashes are regularly repeated at a rate of not less than 160 flashes per minute. |
| 7 | Isophased | A light with all durations of light and darkness equal. |
| 8 | Occulting | A rhythmic light in which the total duration of light in a period is clearly longer than the total duration of darkness and all the eclipses are of equal duration. It may be: - Single-occulting: An occulting light in which an eclipse is regularly repeated. - Group-occulting: An occulting light in which a group of two or more eclipses, which are specified in number, is regularly repeated. - Composite group-occulting: An occulting light in which a sequence of groups of one or more eclipses, which are specified in number, is regularly repeated, and the groups comprise different numbers of eclipses. |
| 9 | Interrupted Quick Flashing | A quick light in which the sequence of flashes is interrupted by regularly repeated eclipses of constant and long duration. |
| 10 | Interrupted Very Quick Flashing | A light in which the very rapid alterations of light and darkness are interrupted at regular intervals by eclipses of long duration. |
| 11 | Interrupted Ultra Quick-Flashing | A light in which the ultra quick flashes (160 or more per minute) are interrupted at regular intervals by eclipses of long duration. |
| 12 | Morse | A rhythmic light in which appearances of light of two clearly different durations are grouped to represent a character or characters in the Morse code. |
| 13 | Fixed and Flash | A rhythmic light in which a fixed light is combined with a flashing light of higher luminous intensity. |
| 14 | Flash and Long-Flash | A rhythmic light in which a flashing light is combined with a long-flashing light of higher luminous intensity. |
| 15 | Occulting and Flash | A rhythmic light in which an occulting light is combined with a flashing light of higher luminous intensity. |
| 16 | Fixed and Long-Flash | A rhythmic light in which a fixed light is combined with a long-flashing light of higher luminous intensity. |
| 17 | Occulting Alternating | An alternating light in which the total duration of light in each period is clearly longer than the total duration of darkness and in which the intervals of darkness (occultations) are all of equal duration. |
| 18 | Long-Flash Alternating | An alternating single-flashing light in which an appearance of light of not less than two seconds duration is regularly repeated. |
| 19 | Flash Alternating | An alternating rhythmic light in which the total duration of light in a period is clearly shorter than the total duration of darkness and all the appearances of light are of equal duration. |
| 20 | Group Alternating | Occulting light in which the occultations are combined in groups, each group including the same number of occultations, and in which the groups are repeated at regular intervals. |
| 25 | Quick-Flash Plus Long-Flash | A rhythmic light in which a group of quick flashes is followed by one or more long flashes in a regularly repeated sequence with a regular periodicity. |
| 26 | Very Quick-Flash Plus Long-Flash | A rhythmic light in which a group of very quick flashes is followed by one or more long flashes in a regularly repeated sequence with a regular periodicity. |
| 27 | Ultra Quick-Flash Plus Long-Flash | A rhythmic light in which a group of ultra quick flashes is followed by one or more long flashes in a regularly repeated sequence with a regular periodicity. |
| 28 | Alternating | A signal light that shows continuously, in any given direction, two or more colours in a regularly repeated sequence with a regular periodicity. |
| 29 | Fixed and Alternating Flashing | A rhythmic light in which a fixed light is combined with a flashing light of higher luminous intensity and different colour. |
| 1 | group-occulting light | An occulting light in which a group of two or more eclipses, which are specified in number, is regularly repeated. |
| 2 | composite group-occulting light | An occulting light in which a sequence of groups of one or more eclipses, which are specified in number, is regularly repeated, and the groups comprise different numbers of eclipses. |
| 3 | group flashing light | A flashing light in which a group of flashes, specified in number, is regularly repeated. |
| 4 | composite group-flashing light | A light similar to a group-flashing light except that successive groups in a period have different numbers of flashes. |
| 5 | group quick light | A quick-flashing light in which a group of two or more flashes, which are specified in number, is regularly repeated. |
| 6 | group very quick light | A very quick-flashing light in which a group of two or more flashes, which are specified in number, is regularly repeated. |

5.35. Interoperability Identifier

Definition: A common unique identifier for entities which describe a single real-world feature, and which is used to identify instances of the feature in end-user systems where the feature may be included in multiple data product types.

CamelCase: interoperabilityIdentifier

Alias:

Value type: URN

Remarks: No remarks.

5.36. Delivery Point

Definition: -

CamelCase: deliveryPoint

Alias:

Value type: text

Remarks: No remarks.

5.37. Postal Code

Definition: -

CamelCase: postalCode

Alias:

Value type: text

Remarks: No remarks.

5.38. language

Definition: -

CamelCase: language

Alias:

Value type: text

Remarks: No remarks.

5.39. Name

Definition: The individual name of a feature.

CamelCase: name

Alias: OBJNAM

Value type: text

Remarks: No remarks.

5.40. City Name

Definition: -

CamelCase: cityName

Alias:

Value type: text

Remarks: No remarks.

5.41. Administrative Division

Definition: -

CamelCase: administrativeDivision

Alias:

Value type: text

Remarks: No remarks.

5.42. Country Name

Definition: -

CamelCase: countryName

Alias:

Value type: text

Remarks: No remarks.

5.43. Scale Minimum

Definition: The minimum scale at which the feature may be used for example for ECDIS presentation.

CamelCase: scaleMinimum

Alias: SCAMIN

Value type: integer

Remarks: The modulus of the scale is indicated, that is 1:1 250 000 is encoded as 1250000.

5.44. Source Date

Definition: The production date of the source; for example the date of measurement.

CamelCase: sourceDate

Alias: SORDAT

Value type: date

Remarks: No remarks.

5.45. Source

Definition: The publication, document, or reference work from which information comes or is acquired.

CamelCase: source

Alias:

Value type: text

Remarks: May be populated with the corresponding paper chart Notice to Mariners numbers, although other references are permitted.

5.46. Pictorial Representation

Definition: The file name of an externally referenced picture file.

CamelCase: pictorialRepresentation

Alias: PICREP

Value type: text

Remarks: The 'pictorial representation' could be a drawing or a photo. The string encodes the file name of an external graphic file (pixel/vector).

5.47. Installation Date

Definition: The date when an item was installed.

CamelCase: installationDate

Alias:

Value type: date

Remarks: No remarks.

5.48. Category of Navigation Line

Definition: Classification of route guidance given to vessels.

CamelCase: categoryOfNavigationLine

Alias: CATNAV

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Clearing Line | A straight line that marks the boundary between a safe and a dangerous area or that passes clear of a navigational danger. |
| 2 | Transit Line | A line passing through one or more fixed marks. |
| 3 | Leading Line Bearing a Recommended Track | A line passing through one or more clearly defined objects, along the path of which a vessel can approach safely up to a certain distance off. |

5.49. Based On Fixed Marks

Definition: A straight route (known as a recommended track, range or leading line), which comprises: a. at least two structures (usually beacons or daymarks) and/or natural features, which may carry lights and/or top-marks. The structures/features are positioned so that when observed to be in line, a vessel can follow a known bearing with safety. (Adapted from International Association of Lighthouse Authorities - IALA Aids to Navigation Guide, 1990); or b. a single structure or natural feature, which may carry lights and/or a topmark, and a specified bearing which can be followed with safety. (S-57 Edition 3.1, Appendix A Chapter 2, Page 2.72, November 2000, as amended).

CamelCase: basedOnFixedMarks

Alias: CATTRK

Value type: boolean

Remarks: No remarks.

5.50. Depth Range Minimum Value

Definition: The minimum (shoalest) value of a depth range.

CamelCase: depthRangeMinimumValue

Alias: DRVAL1

Value type: real

Remarks: Where the area dries, the value is negative.

5.51. Maximal Permitted Draught

Definition: The maximal permitted draught of a vessel or convoy according to the particular article/clause of the applicable law/regulation.

CamelCase: maximalPermittedDraught

Alias: lg_drt

Value type: real

Remarks: No remarks.

5.52. Quality of Vertical Measurement

Definition: The reliability of the value of a sounding.

CamelCase: qualityOfVerticalMeasurement

Alias: QUASOU

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Depth Known | The depth from the chart datum to the seabed (or to the top of a drying feature) is known. |
| 2 | Depth or Least Depth Unknown | The depth from chart datum to the seabed, or the shoalest depth of the feature is unknown. |
| 3 | Doubtful Sounding | A depth that may be less than indicated. |
| 4 | Unreliable Sounding | A depth that is considered to be an unreliable value. |
| 5 | No Bottom Found at Value Shown | Upon investigation the bottom was not found at this depth. |
| 6 | Least Depth Known | The shoalest depth over a feature is of known value. |
| 7 | Least Depth Unknown, Safe Clearance at Value Shown | The least depth over a feature is unknown, but there is considered to be safe clearance at this depth. |
| 8 | Value Reported (Not Surveyed) | Depth value obtained from a report, but not fully surveyed. |
| 9 | Value Reported (Not Confirmed) | Depth value obtained from a report, which it has not been possible to confirm. |
| 10 | Maintained Depth | The depth at which a channel is kept by human influence, usually by dredging. |
| 11 | Not Regularly Maintained | Depths may be altered by human influence, but will not be routinely maintained. |

5.53. Status

Definition: The condition of an object at a given instant in time.

CamelCase: status

Alias: STATUS

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Permanent | Intended to last or function indefinitely. |
| 2 | Occasional | Acting on special occasions; happening irregularly. |
| 3 | Recommended | Presented as worthy of confidence, acceptance, use, etc. |
| 4 | Not in Use | Use has ceased, but the facility still exists intact; disused. |
| 5 | Periodic/Intermittent | Recurring at intervals. |
| 6 | Reserved | Set apart for some specific use. |
| 7 | Temporary | Meant to last only for a time. |
| 8 | Private | Administered by an individual or corporation, rather than a State or a public body. |
| 9 | Mandatory | Compulsory; enforced. |
| 11 | Extinguished | No longer lit. |
| 12 | Illuminated | Lit by flood lights, strip lights, etc. |
| 13 | Historic | Famous in history; of historical interest. |
| 14 | Public | Belonging to, available to, used or shared by, the community as a whole and not restricted to private use. |
| 15 | Synchronized | Occur at a time, coincide in point of time, be contemporary or simultaneous. |
| 16 | Watched | Looked at or observed over a period of time especially so as to be aware of any movement or change. |
| 17 | Unwatched | Usually automatic in operation, without any permanently-stationed personnel to superintend it. |
| 18 | Existence Doubtful | A feature that has been reported but has not been definitely determined to exist. |
| 19 | On Request | When you ask for it. |
| 20 | Drop Away | To become lower in level. |
| 21 | Rising | To become higher in level. |
| 22 | Increasing | Becoming larger in magnitude. |
| 23 | Decreasing | Becoming smaller in magnitude. |
| 24 | Strong | Not easily broken or destroyed. |
| 25 | Good | In a satisfactory condition to use. |
| 26 | Moderately | Fairly but not very. |
| 27 | Poor | Not as good as it could be or should. |
| 28 | Buoyed | Marked by buoys. |
| 29 | Fully Operational | Entire observation platform is operating in accordance with, or exceeding, manufacturer specifications. |
| 30 | Partially Operational | At least one instrument that is part of an observation platform is not operating to manufacturer specification. |
| 31 | Drifting | Floating platform at the mercy of environmental elements, whether intentional or not. |
| 32 | Broken | Fractured or in pieces. |
| 33 | Offline | Observation platform is intentionally not reporting an environmental observation. |
| 34 | Discontinued | Observation station, suite of instruments, or an individual instrument, for a particular location, has been removed and is no longer at the particular location. |
| 35 | Manual Observation | Observations made by a human observer. |
| 36 | Unknown Status | Status of an observation platform, suite of instruments, or individual instrument is not known or unspecified. |
| 37 | Confirmed | Made certain as to truth, accuracy, validity, availability, etc. |
| 38 | Candidate | Item selected for an action. |
| 39 | Under Modification | Item that is in the process of being modified. |
| 41 | Under Removal / Deletion | Item in the process of being removed or deleted. |
| 42 | Removed / Deleted | Item that has been removed or deleted. |
| 43 | Candidate for Modification | Item selected for modification. |

5.54. IsSlatted

Definition: A flag or attribute indicating whether an object has slats, which are flat, narrow strips of material, often used for ventilation or design.

CamelCase: isSlatted

Alias:

Value type: boolean

Remarks: -

5.55. Vertical Accuracy

Definition: The degree to which a vertical measurement is accurate, typically referring to the accuracy of an object's position in the vertical plane (height or depth)

CamelCase: verticalAccuracy

Alias:

Value type: real

Remarks: -

5.56. Peak Intensity

Definition: The maximum luminous intensity of a light during its flash cycle.

CamelCase: peakIntensity

Alias:

Value type: real

Remarks: -

5.57. Technique of Vertical Measurement

Definition: Survey method used to obtain depth information.

CamelCase: techniqueOfVerticalMeasurement

Alias: TECSOU

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Found by Echo Sounder | The depth was measured by using an instrument that determines depth of water by measuring the time interval between emission of a sonic or ultrasonic signal and return of its echo from the bottom. |
| 2 | Found by Side Scan Sonar | The depth was computed from a record produced by active sonar in which fixed acoustic beams are directed into the water perpendicularly to the direction of travel to scan the seabed and generate a record of the seabed configuration. |
| 3 | Found by Multi Beam | The depth was measured by using a wide swath echo sounder that uses multiple beams to measure depths directly below and transverse to the ship's track. |
| 4 | Found by Diver | The depth was determined by a person skilled in the practice of diving. |
| 5 | Found by Lead Line | The depth was measured by using a line, graduated with attached marks and fastened to a sounding lead. |
| 6 | Swept by Wire-Drag | The given area was determined to be free from navigational dangers to a certain depth by towing a buoyed wire at the desired depth by two launches, or a least depth was identified using the same technique. |
| 7 | Found by Laser | The depth was determined by using an instrument that measures distance by emitting timed pulses of laser light and measuring the time between emission and reception of the reflected pulses. |
| 8 | Swept by Vertical Acoustic System | The given area has been swept using a system comprised of multiple echo sounder transducers attached to booms deployed from the survey vessel. |
| 9 | Found by Electromagnetic Sensor | The depth was determined by using an instrument that compares electromagnetic signals. |
| 10 | Photogrammetry | The science or art of obtaining reliable measurements from photographs. |
| 11 | Satellite Imagery | The depth was determined by using instruments placed aboard an artificial satellite. |
| 12 | Found by Levelling | The depth was determined by using levelling techniques to find the elevation of the point relative to a datum. |
| 13 | Swept by Side Scan Sonar | The given area was determined to be free from navigational dangers to a certain depth by towing a side scan sonar. |
| 14 | Computer Generated | The sounding was determined from a bottom model constructed using a computer. |
| 15 | Found by LIDAR | The depth was measured by using an instrument that measures distance by emitting timed pulses of laser light and measuring the time between emission and reception of the reflected pulses. |
| 16 | Synthetic Aperture Radar | A radar with a synthetic aperture antenna which is composed of a large number of elementary transducing elements. The signals are electronically combined into a resulting signal equivalent to that of a single antenna of a given aperture in a given direction. |
| 17 | Hyperspectral Imagery | Term used to describe the imagery derived from subdividing the electromagnetic spectrum into very narrow bandwidths. These narrow bandwidths may be combined with or subtracted from each other in various ways to form images useful in precise terrain or target analysis. |
| 18 | Mechanically Swept | The given area was determined to be free from navigational dangers to a certain depth by towing a line or object below the surface at the desired depth; or least depth(s) and position(s) within an area was identified using the same technique. |

5.58. Traffic Flow

Definition: Direction of vessels passing a reference point.

CamelCase: trafficFlow

Alias: TRAFIC

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Inbound | Traffic flow in a general direction toward a port or similar destination. |
| 2 | Outbound | Traffic flow in a general direction away from a port or similar point of origin. |
| 3 | One-Way | Traffic flow in one general direction only. |
| 4 | Two-Way | Traffic flow in two generally opposite directions. |

5.59. Vertical Datum

Definition: The reference level used for expressing the vertical measurements of points on the earth's surface. Also called datum level, reference plane, levelling datum, datum for sounding reduction, datum for heights.

CamelCase: verticalDatum

Alias: VERDAT Datum Level Reference Plane Levelling Datum Datum for Sounding Reduction Datum for Heights

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Mean Low Water Springs | The average height of the low waters of spring tides. This level is used as a tidal datum in some areas. |
| 2 | Mean Lower Low Water Springs | The average height of lower low water springs at a place. |
| 3 | Mean Sea Level | The average height of the surface of the sea at a tide station for all stages of the tide over a 19-year period, usually determined from hourly height readings measured from a fixed predetermined reference level. |
| 4 | Lowest Low Water | An arbitrary level conforming to the lowest tide observed at a place, or somewhat lower. |
| 5 | Mean Low Water | The average height of all low waters at a place over a 19-year period. |
| 6 | Lowest Low Water Springs | An arbitrary level conforming to the lowest water level observed at a place at spring tides during a period of time shorter than 19 years. |
| 7 | Approximate Mean Low Water Springs | An arbitrary level, usually within 0.3m from that of Mean Low Water Springs (MLWS). |
| 8 | Indian Spring Low Water | An arbitrary tidal datum approximating the level of the mean of the lower low water at spring tides. It was first used in waters surrounding India. |
| 9 | Low Water Springs | An arbitrary level, approximating that of mean low water springs (MLWS). |
| 10 | Approximate Lowest Astronomical Tide | An arbitrary level, usually within 0.3m from that of Lowest Astronomical Tide (LAT). |
| 11 | Nearly Lowest Low Water | An arbitrary level approximating the lowest water level observed at a place, usually equivalent to the Indian Spring Low Water (ISLW). |
| 12 | Mean Lower Low Water | The average height of the lower low waters at a place over a 19-year period. |
| 13 | Low Water | The lowest level reached at a place by the water surface in one oscillation. |
| 14 | Approximate Mean Low Water | An arbitrary level, usually within 0.3m from that of Mean Low Water (MLW). |
| 15 | Approximate Mean Lower Low Water | An arbitrary level, usually within 0.3m from that of Mean Lower Low Water (MLLW). |
| 16 | Mean High Water | The average height of all high waters at a place over a 19-year period. |
| 17 | Mean High Water Springs | The average height of the high waters of spring tides. |
| 18 | High Water | The highest level reached at a place by the water surface in one oscillation. |
| 19 | Approximate Mean Sea Level | An arbitrary level, usually within 0.3m from that of Mean Sea Level (MSL). |
| 20 | High Water Springs | An arbitrary level, approximating that of mean high water springs (MHWS). |
| 21 | Mean Higher High Water | The average height of higher high waters at a place over a 19-year period. |
| 22 | Equinoctial Spring Low Water | The level of low water springs near the time of an equinox. |
| 23 | Lowest Astronomical Tide | The lowest tide level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. |
| 24 | Local Datum | An arbitrary datum defined by a local harbour authority, from which levels and tidal heights are measured by this authority. |
| 25 | International Great Lakes Datum 1985 | A vertical reference system with its zero based on the mean water level at Rimouski/Pointe-au-Père, Quebec, over the period 1970 to 1988. |
| 26 | Mean Water Level | The average of all hourly water levels over the available period of record. |
| 27 | Lower Low Water Large Tide | The average of the lowest low waters, one from each of 19 years of observations. |
| 28 | Higher High Water Large Tide | The average of the highest high waters, one from each of 19 years of observations. |
| 29 | Nearly Highest High Water | An arbitrary level approximating the highest water level observed at a place, usually equivalent to the high water springs. |
| 30 | Highest Astronomical Tide | The highest tidal level which can be predicted to occur under average meteorological conditions and under any combination of astronomical conditions. |
| 31 | Local Low Water Reference Level | Low water reference level of the local area. |
| 32 | Local High Water Reference Level | High water reference level of the local area. |
| 33 | Local Mean Water Reference Level | Mean water reference level of the local area. |
| 34 | Equivalent Height of Water (German GlW) | A low water level which is the result of a defined low water discharge - called "equivalent discharge". |
| 35 | Highest Shipping Height of Water (German HSW) | Upper limit of water levels where navigation is allowed. |
| 36 | Reference Low Water Level According to Danube Commission | The water level at a discharge, which is exceeded 94 % of the year within a period of 30 years. |
| 37 | Highest Shipping Height of Water According to Danube Commission | The water level at a discharge, which is exceeded 1% of the year within a period of 30 years. |
| 38 | Dutch River Low Water Reference Level (OLR) | The water level at a discharge, which is exceeded 95% of the year within a period of 20 years. |
| 39 | Russian Project Water Level | Conditional low water level with established probability. |
| 40 | Russian Normal Backwater Level | Highest water level derived from the upper backwater stream in watercourse or reservoir under the normal operational conditions. |
| 41 | Ohio River Datum | The Ohio River datum. |
| 43 | Dutch High Water Reference Level | Dutch High Water Reference Level. |
| 44 | Baltic Sea Chart Datum 2000 | The datum refers to each Baltic country's realization of the European Vertical Reference System (EVRS) with land-uplift epoch 2000, which is connected to the Normaal Amsterdams Peil (NAP). |
| 45 | Dutch Estuary Low Water Reference Level (OLW) | Dutch Estuary Low Water Reference Level (OLW) |
| 46 | International Great Lakes Datum 2020 | The 2020 update to the International Great Lakes Datum, the official reference system used to measure water level heights in the Great Lakes, connecting channels, and the St. Lawrence River system. |
| 47 | Sea Floor | The bottom of the ocean and seas where there is a generally smooth gentle gradient. Also referred to as sea bed (sometimes seabed or sea-bed), and sea bottom. |
| 48 | Sea Surface | A two-dimensional (in the horizontal plane) field representing the air-sea interface, with high-frequency fluctuations such as wind waves and swell, but not astronomical tides, filtered out. |
| 49 | Hydrographic Zero | A vertical reference near the lowest astronomical tide (LAT), below which the sea level falls only very exceptionally. |

5.60. AtoN Number

Definition: Identifier from a list of Aids to Navigation publication, such as List of Lights.

CamelCase: atoNNumber

Alias:

Value type: text

Remarks: No remarks.

5.61. Buoy Shape

Definition: The principal shape and/or design of a buoy.

CamelCase: buoyShape

Alias: BOYSHP

Value type: enumeration

Remarks: The principal shapes are those recommended in the International Association of Lighthouse Authorities - IALA System.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Conical | The upper part of the body above the water-line, or the greater part of the superstructure, has approximately the shape or the appearance of a pointed cone with the point upwards. |
| 2 | Can | The upper part of the body above the water-line, or the greater part of the superstructure, has the shape of a cylinder, or a truncated cone that approximates to a cylinder, with a flat end uppermost. |
| 3 | Spherical | Shaped like a sphere, which is a body the surface of which is at all points equidistant from the centre. |
| 4 | Pillar | The upper part of the body above the water-line, or the greater part of the superstructure is a narrow vertical structure, pillar or lattice tower. |
| 5 | Spar | The upper part of the body above the water-line, or the greater part of the superstructure, has the form of a pole, or of a very long cylinder, floating upright. |
| 6 | Barrel | The upper part of the body above the water-line, or the greater part of the superstructure, has the form of a barrel or cylinder floating horizontally. |
| 7 | Superbuoy | A very large buoy designed to carry a signal light of high luminous intensity at a high elevation. |
| 8 | Ice Buoy | A specially constructed shuttle shaped buoy which is used in ice conditions. |

5.62. Colour

Definition: The property possessed by an object of producing different sensations on the eye as a result of the way it reflects or emits light.

CamelCase: colour

Alias: COLOUR

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | White | The achromatic object colour of greatest lightness characteristically perceived to belong to objects that reflect diffusely nearly all incident energy throughout the visible spectrum. |
| 2 | Black | The achromatic color of least lightness characteristically perceived to belong to objects that neither reflect nor transmit light. |
| 3 | Red | A color whose hue resembles that of blood or of the ruby or is that of the long-wave extreme of the visible spectrum. |
| 4 | Green | Of the color green. |
| 5 | Blue | A color whose hue is that of the clear sky or that of the portion of the color spectrum lying between green and violet. |
| 6 | Yellow | A color whose hue resembles that of ripe lemons or sunflowers or is that of the portion of the spectrum lying between green and orange. |
| 7 | Grey | Of the color grey. |
| 8 | Brown | Any of a group of colors between red and yellow in hue, of medium to low lightness, and of moderate to low saturation. |
| 9 | Amber | A variable color averaging a dark orange yellow. |
| 10 | Violet | Any of a group of colors of reddish-blue hue, low lightness, and medium saturation. |
| 11 | Orange | Any of a group of colors that are between red and yellow in hue. |
| 12 | Magenta | A deep purplish red. |
| 13 | Pink | Any of a group of colors bluish red to red in hue, of medium to high lightness, and of low to moderate saturation. |

5.63. Category Of Synthetic AIS Aid To Navigation

Definition: A classification of AIS AtoNs that are transmitted electronically and linked to a real-world object but do not physically exist at the broadcast location.

CamelCase: categoryOfSyntheticAISAidtoNavigation

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | predicted | - |
| 2 | monitored | - |

5.64. Category Of Physical AIS Aid To Navigation

Definition: A classification of AIS AtoNs that correspond to an actual, physical Aid to Navigation at a real-world location.

CamelCase: categoryOfPhysicalAISAidToNavigation

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Physical AIS Type 1 | Simple transmission of static, pre-programmed information. |
| 2 | Physical AIS Type 2 | Transmission of dynamic, real-time updated information via connected sensors. |
| 3 | Physical AIS Type 3 | Full two-way communication including transmission, remote control and configuration |

5.65. Colour Pattern

Definition: A regular repeated design containing more than one colour.

CamelCase: colourPattern

Alias: COLPAT

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Horizontal Stripes | Straight bands or stripes of differing colours oriented horizontally. |
| 2 | Vertical Stripes | Straight bands or stripes of differing colours oriented vertically. |
| 3 | Diagonal Stripes | Straight bands or stripes of differing colours oriented diagonally (that is, not horizontally or vertically). |
| 4 | Squared | Often referred to as checker plate, where alternate colours are used to create squares similar to a chess or draught board. The pattern may be straight or diagonal. |
| 5 | Stripes (Direction Unknown) | Straight bands or stripes of differing colours oriented in an unknown direction. |
| 6 | Border Stripe | A band or stripe of colour which is displayed around the outer edge of the feature, which may also form a border to an inner pattern or plain colour. |
| 7 | Single Colour | One solid colour of uniform coverage. |
| 8 | Rectangle | A four-sided shape that is made up of two pairs of parallel lines and that has four right angles, on a different coloured background. |
| 9 | Triangle | A shape that is made up of three lines and three angles, on a different coloured background. |

5.66. Radar Conspicuous

Definition: A feature which returns a strong radar echo.

CamelCase: radarConspicuous

Alias: CONRAD

Value type: boolean

Remarks: No remarks.

5.67. Marks Navigational - System Of

Definition: The system of navigational buoyage a region complies with.

CamelCase: marksNavigationalSystemOf

Alias: MARSYS

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | IALA A | Navigational aids conform to the International Association of Lighthouse Authorities - IALA A system. |
| 2 | IALA B | Navigational aids conform to the International Association of Lighthouse Authorities - IALA B system. |
| 9 | No System | Navigational aids do not conform to any defined system. |
| 10 | Other System | Navigational aids conform to a defined system other than International Association of Lighthouse Authorities - IALA. |
| 11 | Main European Inland Waterway Marking System | Navigational aids as required in international, national or regional regulations that contain the same navigational aids as the European Code for Inland Waterways of UNECE, or if there is no regulation for a waterway, navigational aids as recommended in the European Code for Inland Waterways of UNECE |
| 12 | Russian Inland Waterway Regulations | Navigational aids conform to the Russian inland waterway regulations. |
| 13 | Brazilian National Inland Waterway Regulation | Navigational aids conform to the Brazilian national inland waterway regulation |
| 15 | Paraguay-Parana Waterway - Brazilian Complementary Aids | Navigational aids conform to the Brazilian complementary aids on the Paraguay-Parana waterway. |

5.68. Nature of Construction

Definition: The building's primary construction material.

CamelCase: natureOfConstruction

Alias: NATCON

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Masonry | Constructed of stones or bricks, usually quarried, shaped, and mortared. |
| 2 | Concreted | Constructed of concrete, a material made of sand and gravel that is united by cement into a hardened mass used for roads, foundations, etc. |
| 3 | Loose Boulders | Constructed from large stones or blocks of concrete, often placed loosely for protection against waves or water turbulence. |
| 4 | Hard Surfaced | Constructed with a surface of hard material, usually a term applied to roads surfaced with asphalt or concrete. |
| 5 | Unsurfaced | Constructed with no extra protection, usually a term applied to roads not surfaced with a hard material. |
| 6 | Wooden | Constructed from wood. |
| 7 | Metal | Constructed from metal. |
| 8 | Glass Reinforced Plastic | Constructed from a plastic material strengthened with fibres of glass. |
| 9 | Painted | The application of paint to some other construction or natural feature. |
| 10 | Framework | Constructed from a lattice framework of, often diagonal, intersecting struts. |
| 11 | Latticed | A structure of crossed wooden or metal strips usually arranged to form a diagonal pattern of open spaces between the strips. |
| 12 | Glass | [1] Any artificial or natural substance having similar properties and composition, as fused borax, obsidian, or the like. [2] Something made of such a substance, as a windowpane. |
| 13 | Fiberglass | Constructed from fiberglass. |
| 14 | Plastic | Constructed from plastic. |

5.69. Type of Buoy

Definition: Type equipment used as a buoy in a particular installation.

CamelCase: typeOfBuoy

Alias:

Value type: text

Remarks: Types of light buoy; for example LANBY-100, LS-35, LL-30, LL-28, LL-26, LL-26(M), LL-24, LS-24, LSP-24, LT-10Types of buoy e.g. U-17C(P), U-17S(P), U-17C(S), U-17S(S), UR-17C(P), UR-17S(P), UR-17C(S), UR-17.

5.70. Beacon Shape

Definition: Describes the characteristic geometric form of the beacon.

CamelCase: beaconShape

Alias: BCNSHP

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Stake, Pole, Perch, Post | An elongated wood or metal pole, driven into the ground or seabed, which serves as a navigational aid or a support for a navigational aid. |
| 2 | Withy | A tree without roots stuck or spoiled into the bottom of the sea to serve as a navigational aid. |
| 3 | Beacon Tower | A solid structure of the order of 10 metres in height used as a navigational aid. |
| 4 | Lattice Beacon | A structure consisting of strips of metal or wood crossed or interlaced to form a structure to serve as an aid to navigation or as a support for an aid to navigation. |
| 5 | Pile Beacon | A long heavy timber(s) or section(s) of steel, wood, concrete, etc., forced into the seabed to serve as an aid to navigation or as a support for an aid to navigation. |
| 6 | Cairn | A mound of stones, usually conical or pyramidal, raised as a landmark or to designate a point of importance in surveying. |
| 7 | Buoyant Beacon | A tall spar-like beacon fitted with a permanently submerged buoyancy chamber, the lower end of the body is secured to seabed sinker either by a flexible joint or by a cable under tension. |

5.71. Visual Prominence

Definition: The extent to which a feature, either natural or artificial, is visible from seaward.

CamelCase: visualProminence

Alias: CONVIS

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Visually Conspicuous | Term applied to an object either natural or artificial which is distinctly and notably visible from seaward. |
| 2 | Not Visually Conspicuous | An object that may be visible from seaward, but cannot be used as a fixing mark and is not conspicuous. |
| 3 | Prominent | Objects which are easily identifiable, but do not justify being classed as conspicuous. |

5.72. Height

Definition: The value of the vertical distance to the highest point of the feature, measured from a specified vertical datum.

CamelCase: height

Alias: HEIGHT

Value type: real

Remarks: Height must not be used for floating objects.

5.73. Vertical Length

Definition: The total vertical length of a feature.

CamelCase: verticalLength

Alias: VERLEN

Value type: real

Remarks: For floating objects: the vertical distance from the surface of water to the highest point of that object. For fixed objects: the vertical distance from seabed or ground to the highest point of that object. For objects on top of other objects: the vertical distance from the lowest to the highest point of that object. Vertical length measurements do not require a datum.

5.74. Category of Landmark

Definition: Classification of prominent cultural and natural features in the landscape.

CamelCase: categoryOfLandmark

Alias: CATLMK

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Cairn | A mound of stones, usually conical or pyramidal, raised as a landmark or to designate a point of importance in surveying. |
| 2 | Cemetery | A site and associated structures devoted to the burial of the dead. |
| 3 | Chimney | A vertical structure containing a passage or flue for discharging smoke and gases of combustion. |
| 4 | Dish Aerial | A parabolic aerial for the receipt and transmission of high frequency radio signals. |
| 5 | Flagstaff | A staff or pole on which flags are raised. |
| 6 | Flare Stack | A tall structure used for burning-off waste oil or gas. |
| 7 | Mast | A relatively tall structure usually held vertical by guy lines. |
| 8 | Windsock | A tapered fabric sleeve mounted so as to catch and swing with the wind, thus indicating the wind direction. |
| 9 | Monument | A structure erected and/or maintained as a memorial to a person and/or event. |
| 10 | Column/Pillar | A cylindrical or slightly tapering body of considerably greater length than diameter erected vertically. |
| 11 | Memorial Plaque | A slab of metal, usually ornamented, erected as a memorial to a person or event. |
| 12 | Obelisk | A tapering shaft usually of stone or concrete, square or rectangular in section, with a pyramidal apex. |
| 13 | Statue | A representation of a living being, sculptured, moulded, or cast in a variety of materials (for example: marble, metal, or plaster). |
| 14 | Cross | A monument, or other structure in form of a cross. |
| 15 | Dome | A landmark comprising a hemispherical or spheroidal shaped structure. |
| 16 | Radar Scanner | A device used for directing a radar beam through a search pattern. |
| 17 | Tower | A relatively tall, narrow structure that may either stand alone or may form part of another structure. |
| 18 | Windmill | A system of vanes attached to a tower and driven by wind (excluding wind turbines). |
| 19 | Windmotor | A modern structure for the use of wind power. |
| 20 | Spire/Minaret | A tall conical or pyramid-shaped structure often built on the roof or tower of a building, especially a church or mosque. |
| 21 | Large Rock or Boulder on Land | An isolated rocky formation or a single large stone. |
| 22 | Triangulation Mark | A recoverable point on the earth, whose geographic position has been determined by angular methods with geodetic instruments. A triangulation point is a selected point, which has been marked with a station mark, or it is a conspicuous natural or artificial feature. |
| 23 | Boundary Mark | A marker identifying the location of a surveyed boundary line. |
| 24 | Observation Wheel | Wheels with passenger cars mounted external to the rim and independently rotated by electric motors. |
| 25 | Torii | A form of decorative gateway or portal, consisting of two upright wooden posts connected at the top by two horizontal crosspieces, commonly found at the entrance to Shinto temples. |
| 26 | Bridge | (1) An elevated structure extending across or over the weather deck of a vessel, or part of such a structure. The term is sometimes modified to indicate the intended use, such as navigating bridge or signal bridge. (2) A structure erected over a depression or an obstacle such as a body of water, railroad, etc., to provide a roadway for vehicles or pedestrians. |
| 27 | Dam | A barrier to check or confine anything in motion; particularly one constructed to hold back water and raise its level to form a reservoir, or to prevent flooding. |

5.75. Function

Definition: A specific role that describes a feature.

CamelCase: function

Alias: FUNCTN

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 2 | Harbour-Masters Office | A local official who has charge of mooring and berthing of vessels, collecting harbour fees, etc. |
| 3 | Customs Office | Serves as a government office where customs duties are collected, the flow of goods are regulated and restrictions enforced, and shipments or vehicles are cleared for entering or leaving a country. |
| 4 | Health Office | The office which is charged with the administration of health laws and sanitary inspections. |
| 5 | Hospital | An institution or establishment providing medical or surgical treatment for the ill or wounded. |
| 6 | Post Office | The public department, agency or organisation responsible primarily for the collection, transmission and distribution of mail. |
| 7 | Hotel | An establishment, especially of a comfortable or luxurious kind, where paying visitors are provided with accommodation, meals and other services. |
| 8 | Railway Station | A building with platforms where trains arrive, load, discharge and depart. |
| 9 | Police Station | The headquarters of a local police force and that is where those under arrest are first charged. |
| 10 | Water-Police Station | The headquarters of a local water-police force. |
| 11 | Pilot Office | The office or headquarters of pilots; the place where the services of a pilot may be obtained. |
| 12 | Pilot Lookout | A distinctive structure or place on shore from which personnel keep watch upon events at sea or along the coast. |
| 13 | Bank Office | An office for custody, deposit, loan, exchange or issue of money. |
| 14 | Headquarters for District Control | The quarters of an executive officer (director, manager, etc.) with responsibility for an administrative area. |
| 15 | Transit Shed/Warehouse | A building or part of a building for storage of wares or goods. |
| 16 | Factory | A building or buildings with equipment for manufacturing; a workshop. |
| 17 | Power Station | A stationary plant containing apparatus for large scale conversion of some form of energy (such as hydraulic, steam, chemical or nuclear energy) into electrical energy. |
| 18 | Administrative | A building for the management of affairs. |
| 19 | Educational Facility | An establishment for teaching and learning (for example school, college, university, etc). |
| 20 | Church | A building for public Christian worship. |
| 21 | Chapel | A place for Christian worship other than a parish, cathedral or church, especially one attached to a private house or institution. |
| 22 | Temple | A building for public Jewish worship. |
| 23 | Pagoda | A Hindu or Buddhist temple or sacred building. |
| 24 | Shinto Shrine | A building for public Shinto worship. |
| 25 | Buddhist Temple | A building for public Buddhist worship. |
| 26 | Mosque | A Muslim place of worship. |
| 27 | Marabout | A shrine marking the burial place of a Muslim holy man. |
| 28 | Lookout | Keeping a watch upon events at sea or along the coast. |
| 29 | Communication | Transmitting and/or receiving electronic communication signals. |
| 30 | Television | A system for reproducing on a screen visual images transmitted (usually with sound) by radio signals. |
| 31 | Radio | Transmitting and/or receiving radio-frequency electromagnetic waves as a means of communication. |
| 32 | Radar | A method, system or technique of using beamed, reflected, and timed radio waves for detecting, locating, or tracking objects, and for measuring altitudes. |
| 33 | Light Support | A structure serving as a support for one or more lights. |
| 34 | Microwave | Broadcasting and receiving signals using microwaves. |
| 35 | Cooling | Generation of chilled liquid and/or gas for cooling purposes. |
| 36 | Observation | A place from which the surroundings can be observed but at which a watch is not habitually maintained. |
| 37 | Timeball | A visual time signal in the form of a ball. |
| 38 | Clock | Instrument for measuring time and recording hours. |
| 39 | Control | Used to control the flow of traffic within a specified range of an installation. |
| 40 | Airship Mooring | Equipment or structure to secure an airship. |
| 41 | Stadium | An arena for holding and viewing events. |
| 42 | Bus Station | A building where buses and coaches regularly stop to take on and/or let off passengers, especially for long-distance travel. |
| 43 | Passenger Terminal Building | A building within a terminal for the loading and unloading of passengers. |
| 44 | Sea Rescue Control | A unit responsible for promoting efficient organization of search and rescue services and for coordinating the conduct of search and rescue operations within a search and rescue region. |
| 45 | Observatory | A building designed and equipped for making observations of astronomical, meteorological, or other natural phenomena. |
| 46 | Ore Crusher | A building or structure used to crush ore. |
| 47 | Boathouse | A building or shed, usually built partly over water, for sheltering a boat or boats. |
| 48 | Pumping Station | A facility to move solids, liquids or gases by means of pressure or suction. |
| 49 | Roof Above Navigable Water | A roof that is extending above navigable water, e.g. to protect open cargo holds from rain during loading and unloading. Depending on the vertical clearance vessels can pass under the roof above navigable water. |
| 50 | Building Above Navigable Water | The part of a building on land that is extending above navigable water. Depending on the vertical clearance vessels can pass under the building above navigable water. |

5.76. Category of Special Purpose Mark

Definition: Classification of an aid to navigation which signifies some special purpose.

CamelCase: categoryOfSpecialPurposeMark

Alias: CATSPM

Value type: enumeration

Remarks: A mark may be a beacon, a buoy, a signpost or may take another form.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Firing Danger Area Mark | A mark used to indicate a firing danger area, usually at sea. |
| 2 | Target Mark | Any object toward which something is directed. The distinctive marking or instrumentation of a ground point to aid its identification on a photograph. |
| 3 | Marker Ship Mark | A mark marking the position of a ship which is used as a target during some military exercise. |
| 4 | Degaussing Range Mark | A mark used to indicate a degaussing range. |
| 5 | Barge Mark | A mark of relevance to barges. |
| 6 | Cable Mark | A mark used to indicate the position of submarine cables or the point at which they run on to the land. |
| 7 | Spoil Ground Mark | A mark used to indicate the limit of a spoil ground. |
| 8 | Outfall Mark | A mark used to indicate the position of an outfall or the point at which it leaves the land. |
| 9 | ODAS | Ocean Data Acquisition System. |
| 10 | Recording Mark | A mark used to record data for scientific purposes. |
| 11 | Seaplane Anchorage Mark | A mark used to indicate a seaplane anchorage. |
| 12 | Recreation Zone Mark | A mark used to indicate a recreation zone. |
| 13 | Private Mark | A privately maintained mark. |
| 14 | Mooring Mark | A mark indicating a mooring or moorings. |
| 15 | LANBY | A large buoy designed to take the place of a lightship where construction of an offshore light station is not feasible. |
| 16 | Leading Mark | Aids to navigation or other indicators so located as to indicate the path to be followed. Leading marks identify a leading line when they are in transit. |
| 17 | Measured Distance Mark | A mark forming part of a transit indicating one end of a measured distance. |
| 18 | Notice Mark | A notice board or sign indicating information to the mariner. |
| 19 | TSS Mark | A mark indicating a Traffic Separation Scheme. |
| 20 | Anchoring Prohibited Mark | A mark indicating an anchoring prohibited area. |
| 21 | Berthing Prohibited Mark | A mark indicating that berthing is prohibited. |
| 22 | Overtaking Prohibited Mark | A mark indicating that overtaking is prohibited. |
| 23 | Two-Way Traffic Prohibited Mark | A mark indicating a one-way route. |
| 24 | Reduced Wake Mark | A mark indicating that vessels must not generate excessive wake. |
| 25 | Speed Limit Mark | A mark indicating that a speed limit applies. |
| 26 | Stop Mark | A mark indicating the place where the bow of a ship must stop when traffic lights show red. |
| 27 | General Warning Mark | A mark indicating that special caution must be exercised in the vicinity of the mark. |
| 28 | Sound Ship's Siren Mark | A mark indicating that a ship should sound its siren or horn. |
| 29 | Restricted Vertical Clearance Mark | A mark indicating the minimum vertical space available for passage. |
| 30 | Maximum Vessel's Draught Mark | A mark indicating the maximum draught of vessel permitted. |
| 31 | Restricted Horizontal Clearance Mark | A mark indicating the minimum horizontal space available for passage. |
| 32 | Strong Current Warning Mark | A mark warning of strong currents. |
| 33 | Berthing Permitted Mark | A mark indicating that berthing is allowed. |
| 34 | Overhead Power Cable Mark | A mark indicating an overhead power cable. |
| 35 | Channel Edge Gradient Mark | A mark indicating the gradient of the slope of a dredge channel edge. |
| 36 | Telephone Mark | A mark indicating the presence of a telephone. |
| 37 | Ferry Crossing Mark | A mark indicating that a ferry route crosses the ship route; often used with a 'sound ship's siren' mark. |
| 39 | Pipeline Mark | A mark used to indicate the position of submarine pipelines or the point at which they run on to the land. |
| 40 | Anchorage Mark | A mark indicating an anchorage area. |
| 41 | Clearing Mark | A mark used to indicate a clearing line. |
| 42 | Control Mark | A mark indicating the location at which a restriction or requirement exists. |
| 43 | Diving Mark | A mark indicating that diving may take place in the vicinity. |
| 44 | Refuge Beacon | A mark providing or indicating a place of safety. |
| 45 | Foul Ground Mark | A mark indicating a foul ground. |
| 46 | Yachting Mark | A mark installed for use by yachtsmen. |
| 47 | Heliport Mark | A mark indicating an area where helicopters may land. |
| 48 | GNSS Mark | A mark indicating a location at which a GNSS position has been accurately determined. |
| 49 | Seaplane Landing Mark | A mark indicating an area where seaplanes land. |
| 50 | Entry Prohibited Mark | A mark indicating that entry is prohibited. |
| 51 | Work in Progress Mark | A mark indicating that work (generally construction) is in progress. |
| 52 | Mark With Unknown Purpose | A mark whose detailed characteristics are unknown. |
| 53 | Wellhead Mark | A mark indicating a borehole that produces or is capable of producing oil or natural gas. |
| 54 | Channel Separation Mark | A mark indicating the point at which a channel divides separately into two channels. |
| 55 | Marine Farm Mark | A mark indicating the existence of a fish, mussel, oyster or pearl farm/culture. |
| 56 | Artificial Reef Mark | A mark indicating the existence or the extent of an artificial reef. |
| 57 | Ice Mark | A mark, used year round, that may be submerged when ice passes through the area. |
| 58 | Nature Reserve Mark | A mark used to define the boundary of a nature reserve. |
| 59 | Fish Aggregating Device | A fish aggregating (or aggregation) device (FAD) is a man-made object used to attract ocean going pelagic fish such as marlin, tuna and mahi-mahi (dolphin fish). They usually consist of buoys or floats tethered to the ocean floor with concrete blocks or adrift. |
| 60 | Wreck Mark | A mark used to indicate the existence of a wreck. |
| 61 | Customs Mark | A mark used to indicate the existence of a customs checkpoint. |
| 62 | Causeway Mark | A mark used to indicate the existence of a causeway. |
| 63 | Wave Recorder | A surface following buoy used to measure wave activity. |
| 64 | Jetski Prohibited | A mark indicating a jetski prohibited area. |

5.77. Topmark/Daymark Shape

Definition: The shape a topmark or daymark exhibits.

CamelCase: topmarkDaymarkShape

Alias: TOPSHP

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Cone (Point Up) | Is where the vertex points up. |
| 2 | Cone (Point Down) | Is where the vertex points down. |
| 3 | Sphere | A curved surface all points of which are equidistant from a fixed point within, called the centre. |
| 4 | 2 Spheres | Two spheres, one above the other. Two black spheres are commonly used as an International Association of Lighthouse Authorities - IALA topmark (isolated danger). |
| 5 | Cylinder | A solid geometrical figure generated by straight lines fixed in direction and describing with one of point a closed curve, especially a circle (in which case the figure is circular cylinder, its ends being parallel circles). |
| 6 | Board | Usually of rectangular shape, made from timber or metal and used to provide a contrast with the natural background of a daymark. The actual daymark is often painted on to this board. |
| 7 | X-Shaped | Having a shape or a cross-section like the capital letter X. |
| 8 | Upright Cross | A cross with one vertical member and one horizontal member; that is, similar in shape to the character '+'. |
| 9 | Cube (Point Up) | A cube standing on one of its vertexes. A cube is a solid contained by six equal squares, a regular hexahedron. |
| 10 | 2 Cones (Point to Point) | 2 cones, one above the other, with their vertices together in the centre. |
| 11 | 2 Cones (Base to Base) | 2 cones, one above the other, with their bases together in the centre and their vertices pointing up and down. |
| 12 | Rhombus | A plane figure having four equal sides and equal opposite angles (two acute and two obtuse); an oblique equilateral parallelogram. |
| 13 | 2 Cones (Points Upward) | 2 cones, one above the other, with their vertices pointing up. |
| 14 | 2 Cones (Points Downward) | 2 cones, one above the other, with their vertices pointing down. |
| 15 | Besom (Point Up) | A bundle of rods or twigs. A besom, point up is where the thicker (untied) end of the besom is at the bottom. |
| 16 | Besom (Point Down) | A bundle of rods or twigs. A besom, point down is where the thinner (tied) end of the besom is at the bottom. |
| 17 | Flag | A flag mounted on a short pole. |
| 18 | Sphere Over a Rhombus | A sphere located above a rhombus. |
| 19 | Square | A plane figure with four right angles and four equal straight sides. |
| 20 | Rectangle (Horizontal) | A horizontal rectangle is where the two longer opposite sides are standing horizontally. |
| 21 | Rectangle (Vertical) | A rectangle is a plane figure with four right angles and four straight sides, opposite sides being parallel and equal in length. A vertical rectangle is where the two longer opposite sides are standing vertically. |
| 22 | Trapezium (Up) | A quadrilateral having one pair of opposite sides parallel, and which stands on its longer parallel side. |
| 23 | Trapezium (Down) | A quadrilateral having one pair of opposite sides parallel, and which stands on its shorter parallel side. |
| 24 | Triangle (Point Up) | A figure having three angles and three sides, and which has a vertex at the top. |
| 25 | Triangle (Point Down) | A figure having three angles and three sides, and which has a side at the top. |
| 26 | Circle | A perfectly round plane figure whose circumference is everywhere equidistant from its centre. |
| 27 | Two Upright Crosses (One Over the Other) | Two upright crosses, generally vertically disposed one above the other. |
| 28 | T-Shape | Having a shape like the capital letter T. |
| 29 | Triangle Pointing Up Over a Circle | A triangle, vertex uppermost, located above a circle. |
| 30 | Upright Cross Over a Circle | An upright cross located above a circle. |
| 31 | Rhombus Over a Circle | A rhombus located above a circle. |
| 32 | Circle Over a Triangle Pointing Up | A circle located over a triangle, vertex uppermost. |
| 33 | Other Shape (See Shape Information) | An uncommon and/or non-standardized shape as textually described using an associated attribute. |
| 34 | Tubular | Having the form of or consisting of a tube. |

5.78. Category of Fog Signal

Definition: Classification of the various means of generating the fog signal.

CamelCase: categoryOfFogSignal

Alias: CATFOG

Value type: enumeration

Remarks: The classification 'horn' is the generic term for fog signals 'nautophone', 'reed' and 'tyfon'.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Explosive | A signal produced by the firing of explosive charges. |
| 2 | Diaphone | A diaphone uses compressed air and generally emits a powerful low-pitched sound, which often concludes with a brief sound of suddenly lowered pitch, termed the 'grunt'. |
| 3 | Siren | A type of fog signal apparatus which produces sound by virtue of the passage of air through slots or holes in a revolving disk. |
| 4 | Nautophone | A horn having a diaphragm oscillated by electricity. |
| 5 | Reed | [1] A reed uses compressed air and emits a weak, high pitched sound. [2] Any of various water or marsh plants with a firm stem. (Concise Oxford English Dictionary) |
| 6 | Tyfon | A diaphragm horn which operates under the influence of compressed air or steam. |
| 7 | Bell | A ringing sound with a short range. |
| 8 | Whistle | A distinctive sound made by a jet of air passing through an orifice. The apparatus may be operated automatically, by hand or by air being forced up a tube by waves acting on a buoy. |
| 9 | Gong | A sound produced by vibration of a disc when struck. |
| 10 | Horn | A horn uses compressed air or electricity to vibrate a diaphragm and exists in a variety of types which differ greatly in their sound and power. |

5.79. Category of Radar Transponder Beacon

Definition: Classification of radar transponder beacon based on functionality.

CamelCase: categoryOfRadarTransponderBeacon

Alias: CATRTB

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Ramark, Radar Beacon Transmitting Continuously | A radar marker beacon which continuously transmits a signal appearing as a radial line on a radar screen, the line indicating the direction of the beacon. Ramarks are intended primarily for marine use. The name 'ramark' is derived from the words radar marker. |
| 2 | Racon, Radar Transponder Beacon | A radar beacon which returns a coded signal which provides identification of the beacon, as well as range and bearing. The range and bearing are indicated by the location of the first character received on the radar screen. The name 'racon' is derived from the words radar beacon. |
| 3 | Leading Racon/Radar Transponder Beacon | A radar beacon that may be used (in conjunction with at least one other radar beacon) to indicate a leading line. |

5.80. Signal Group

Definition: The number of signals, the combination of signals or the morse character(s) within one period of full sequence.

CamelCase: signalGroup

Alias: SIGGRP

Value type: text

Remarks: No remarks.

5.81. Signal Period

Definition: 	The time occupied by an entire cycle of intervals of light and eclipse.

CamelCase: signalPeriod

Alias:

Value type: real

Remarks: No remarks.

5.82. Value of Nominal Range

Definition: The luminous range of a light in a homogenous atmosphere in which the meteorological visibility is 10 sea miles.

CamelCase: valueOfNominalRange

Alias: VALNMR

Value type: real

Remarks: No remarks.

5.83. Category of Radio Station

Definition: Classification of radio services offered by a radio station.

CamelCase: categoryOfRadioStation

Alias: CATROS

Value type: enumeration

Remarks: A radiobeacon is a radio transmitter which emits a distinctive or characteristic signal on which a bearing may be taken.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Circular (Non-Directional) Marine or Aero-Marine Radiobeacon | A radio station which need not necessarily be manned, the emissions of which, radiated around the horizon, enable its bearing to be determined by means of the radio direction finder of a ship. |
| 2 | Directional Radiobeacon | A special type of radiobeacon station the emissions of which are intended to provide a definite track for guidance. |
| 3 | Rotating Pattern Radiobeacon | A special type of radiobeacon station emitting a beam of waves to which a uniform turning movement is given, the bearing of the station being determined by means of an ordinary listening receiver and a stop watch. Also referred to as a rotating loop radiobeacon. |
| 4 | Consol Beacon | A type of long range position fixing beacon. |
| 5 | Radio Direction-Finding Station | A radio station intended to determine only the direction of other stations by means of transmission from the latter. |
| 6 | Coast Radio Station Providing QTG Service | A radio station which is prepared to provide QTG service; that is to say, to transmit upon request from a ship a radio signal, the bearing of which can be taken by that ship. |
| 7 | Aeronautical Radiobeacon | A radio beacon designed for aeronautical use. |
| 8 | Decca | The Decca Navigator System is a high accuracy, short to medium range radio navigational aid intended for coastal and landfall navigation. |
| 9 | Loran C | A low frequency electronic position fixing system using pulsed transmissions at 100 Khz. |
| 10 | Differential GNSS | Differential GNSS is implemented by placing a GNSS monitor receiver at a precisely known location. Instead of computing a navigation fix, the monitor determines the range error to every GNSS satellite it can track. These ranging errors are then transmitted to local users where they are applied as corrections before computing the navigation result. |
| 11 | Toran | An electronic position fixing system used mainly by aircraft. |
| 12 | Omega | A long-range radio navigational aid which operates within the VLF frequency band. The system comprises eight land based stations. |
| 13 | Syledis | A ranging position fixing system operating at 420-450 MHz over a range of up to 400 Km. |
| 14 | Chaika | A low frequency electronic position fixing system using pulsed transmissions at 100 Khz. |
| 19 | Radio Telephone Station | The equipment needed at one station to carry on two way voice communication by radio waves only. |
| 20 | AIS Base Station | An AIS shore station for use by competent authorities to provide AIS service, manage the data link and enable effective ship to shore / shore to ship transmission of information. |

5.84. Effective Intensity

Definition: The luminous intensity of a fictitious juxtaposed steady-burning point light source that would appear to exhibit a luminosity equal to that of the rhythmic point light source it describes. The apparent reduction in intensity of the rhythmic light is subjective and is due to the nature of the response of the eye of the observer.

CamelCase: effectiveIntensity

Alias:

Value type: real

Remarks: -

5.85. Estimated Range of Transmission

Definition: The estimated range of a non-optical electromagnetic transmission.

CamelCase: estimatedRangeOfTransmission

Alias: ESTRNG

Value type: real

Remarks: The estimated range (distance) assumes 'in vacuo' transmission and a standard antenna height of 5 metres. Thus it gives a hint to the mariner whether they are likely to receive transmission at a certain distance from an object.

5.86. Light Visibility

Definition: The specific visibility of a light, with respect to the light's intensity and ease of recognition.

CamelCase: lightVisibility

Alias: LITVIS

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | High Intensity | Non-marine lights with a higher power than marine lights and visible from well off shore (often 'Aero' lights). |
| 2 | Low Intensity | Non-marine lights with lower power than marine lights. |
| 3 | Faint | A decrease in the apparent intensity of a light which may occur in the case of partial obstructions. |
| 4 | Intensified | A light in a sector is intensified (that is, has longer range than other sectors). |
| 5 | Unintensified | A light in a sector is unintensified (that is, has shorter range than other sectors). |
| 6 | Visibility Deliberately Restricted | A light sector is deliberately reduced in intensity, for example to reduce its effect on a built-up area. |
| 7 | Obscured | Said of the arc of a light sector designated by its limiting bearings in which the light is not visible from seaward. |
| 8 | Partially Obscured | This value specifies that parts of the sector are obscured. |
| 9 | Visible in Line of Range | Lights that must be in line to be visible. |

5.87. Exhibition Condition of Light

Definition: The outward display of the light.

CamelCase: exhibitionConditionOfLight

Alias: EXCLIT

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Light Shown Without Change of Character | A light shown throughout the 24 hours without change of character. |
| 2 | Daytime Light | A light which is only exhibited by day. |
| 3 | Fog Light | A light which is exhibited in fog or conditions of reduced visibility. |
| 4 | Night Light | A light which is only exhibited at night. |

5.88. Flare Bearing

Definition: The bearing about which the light flare symbol is rotated to be displayed in ECDIS.

CamelCase: flareBearing

Alias:

Value type: integer

Remarks: The initial flare bearing is calculated by ENC production software systems.  The attribute flare bearing may also be populated manually to cartographically align the light flare along, for example, a transit or leading line (noting that the in such cases the bearing to be encoded will be the reciprocal (+/- 180 degrees) of the bearing encoded for the navigational line).

5.89. Signal Generation

Definition: The mechanism used to generate a fog or light signal.

CamelCase: signalGeneration

Alias: SIGGEN

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Automatically | Signal generation is initiated by a self regulating mechanism such as a timer or light sensor. |
| 2 | By Wave Action | The signal is generated by the motion of the sea surface such as a bell in a buoy. |
| 3 | By Hand | The signal is generated by a manually operated mechanism such as a hand cranked siren. |
| 4 | By Wind | The signal is generated by the motion of air such as a wind driven whistle. |
| 5 | Radio Activated | Activated by radio signal. |
| 6 | Call Activated | Activated by making a call to a manned station. |

5.90. Major Light

Definition: A statement expressing if a light is considered to be a major light in terms of ECDIS display in a particular area.

CamelCase: majorLight

Alias:

Value type: boolean

Remarks: Major light is only intended to provide an indication to the ECDIS that the light is considered to be an important light in terms of its display. As such this is a cartographic attribute to aid the compiler in determining the most appropriate display for a light; it is not intended to be used as a formal classification method for lights.

5.91. Category of Light

Definition: Classification of different light types.

CamelCase: categoryOfLight

Alias: CATLIT

Value type: enumeration

Remarks: All lights are considered to be marine lights unless the category of light indicates otherwise.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Directional Function | A light illuminating a sector of very narrow angle and intended to mark a direction to follow. |
| 4 | Leading Light | A light associated with other lights so as to form a leading line to be followed. |
| 5 | Aero Light | An aero light is established for aeronautical navigation and may be of higher power than marine lights and visible from well offshore. |
| 6 | Air Obstruction Light | A light marking an obstacle which constitutes a danger to air navigation. |
| 8 | Flood Light | A broad beam light used to illuminate a structure or area. |
| 9 | Strip Light | A light whose source has a linear form generally horizontal, which can reach a length of several metres. |
| 10 | Subsidiary Light | A light placed on or near the support of a main light and having a special use in navigation. |
| 11 | Spotlight | A powerful light focused so as to illuminate a small area. |
| 12 | Front | Term used with leading lights to describe the position of the light on the lead as viewed from seaward. |
| 13 | Rear | Term used with leading lights to describe the position of the light on the lead as viewed from seaward. |
| 14 | Lower | Term used with leading lights to describe the position of the light on the lead as viewed from seaward. |
| 15 | Upper | Term used with leading lights to describe the position of the light on the lead as viewed from seaward. |
| 17 | Emergency | A light available as a backup to a main light which will be illuminated should the main light fail. |
| 18 | Bearing Light | A light which enables its approximate bearing to be obtained without the use of a compass. |
| 19 | Horizontally Disposed | A group of lights of identical character and almost identical position, that are disposed horizontally. |
| 20 | Vertically Disposed | A group of lights of identical character and almost identical position, that are disposed vertically. |

5.92. Category of Offshore Platform

Definition: Classification of an offshore raised structure.

CamelCase: categoryOfOffshorePlatform

Alias: CATOFP

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Oil Rig | A temporary mobile structure, either fixed or floating, used in the exploration stages of oil and gas fields. |
| 2 | Production Platform | A term used to indicate a permanent offshore structure equipped to control the flow of oil or gas. It does not include entirely submarine structures. |
| 3 | Observation/Research Platform | A platform from which one's surroundings or events can be observed, noted or recorded such as for scientific study. |
| 4 | Articulated Loading Platform | A metal lattice tower, buoyant at one end and attached at the other by a universal joint to a concrete filled base on the seabed. The platform may be fitted with a helicopter platform, emergency accommodation and hawser/hose retrieval. |
| 5 | Single Anchor Leg Mooring | A rigid frame or tube with a buoyancy device at its upper end, secured at its lower end to a universal joint on a large steel or concrete base resting on the seabed, and at its upper end to a mooring buoy by a chain or wire. |
| 6 | Mooring Tower | A platform secured to the seabed and surmounted by a turntable to which ships moor. |
| 7 | Artificial Island | A man-made structure usually built for the exploration or exploitation of marine resources, marine scientific research, tidal observations, etc. |
| 8 | Floating Production, Storage and Off-Loading Vessel | An offshore facility consisting of a moored tanker/barge by which the product is extracted, stored and exported. |
| 9 | Accommodation Platform | A platform used primarily for eating, sleeping and recreation purposes. |
| 10 | Navigation, Communication and Control Buoy | A floating structure with control room, power and storage facilities, attached to the seabed by a flexible pipeline and cables. |
| 11 | Floating Oil Tank | A floating structure, anchored to the seabed, for storing oil. |

5.93. Condition

Definition: The various conditions of buildings and other constructions.

CamelCase: condition

Alias: CONDTN

Value type: enumeration

Remarks: The default 'condition' should be considered to be completed, undamaged and working normally.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Under Construction | Being built but not yet capable of function. |
| 2 | Ruined | A structure in a decayed or deteriorated condition resulting from neglect or disuse, or a damaged structure in need of repair. |
| 3 | Under Reclamation | An area of the sea, a lake or the navigable part of a river that is being reclaimed as land, usually by the dumping of earth and other material. |
| 4 | Wingless | A windmill or wind turbine from which the vanes or turbine blades are missing. |
| 5 | Planned Construction | Detailed planning has been completed but construction has not been initiated. |

5.94. Category of Pile

Definition: Classification of pile, driven into the earth as a foundation or support for a structure.

CamelCase: categoryOfPile

Alias: CATPLE

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Stake | An elongated wood or metal pole embedded in the seabed to serve as a marker or support. |
| 3 | Post | A vertical piece of timber, metal or concrete forced into the earth or seabed. |
| 4 | Tripodal | A single structure comprising 3 or more piles held together (sections of heavy timber, steel or concrete), and forced into the earth or seabed. |
| 5 | Piling | A number of piles, usually in a straight line, and usually connected or bolted together. |
| 6 | Area of Piles | A number of piles, usually in a straight line, but not connected by structural members. |
| 7 | Pipe | A vertical hollow cylinder of metal, wood, or other material forced into the earth or seabed. |
| 8 | Mooring Post | A post where to which something (such as a craft) can be moored. |

5.95. Building Shape

Definition: The specific shape of the building.

CamelCase: buildingShape

Alias: BUISHP

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 5 | High-Rise Building | A building having many storeys. |
| 6 | Pyramid | A polyhedron of which one face is a polygon of any number of sides, and the other faces are triangles with a common vertex. |
| 7 | Cylindrical | Shaped like a cylinder, which is a solid geometrical figure generated by straight lines fixed in direction and describing with one of its points a closed curve, especially a circle. |
| 8 | Spherical | Shaped like a sphere, which is a body the surface of which is at all points equidistant from the centre. |
| 9 | Cubic | A shape the sides of which are six equal squares; a regular hexahedron. |

5.96. Category of Silo/Tank

Definition: Classification based on the product for which a silo or tank is used.

CamelCase: categoryOfSiloTank

Alias: CATSIL

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Silo in General | A large storage structure used for storing loose materials. |
| 2 | Tank in General | A fixed structure for storing liquids. |
| 3 | Grain Elevator | A storage building for grain. Usually a tall frame, metal or concrete structure with an especially compartmented interior. |
| 4 | Water Tower | A tower supporting an elevated storage tank of water. |

5.97. Category of Lateral Mark

Definition: Classification of lateral marks in the IALA Buoyage System.

CamelCase: categoryOfLateralMark

Alias: CATLAM

Value type: enumeration

Remarks: There are two international buoyage regions, A and B, between which lateral marks differ. When top-marks, retro reflectors and/or lights are fitted to these marks, they are encoded as separate features.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Port-Hand Lateral Mark | Indicates the port boundary of a navigational channel or suggested route when proceeding in the "conventional direction of buoyage". |
| 2 | Starboard-Hand Lateral Mark | Indicates the starboard boundary of a navigational channel or suggested route when proceeding in the "conventional direction of buoyage". |
| 3 | Preferred Channel to Starboard Lateral Mark | At a point where a channel divides, when proceeding in the "conventional direction of buoyage", the preferred channel (or primary route) is indicated by a modified port-hand lateral mark. |
| 4 | Preferred Channel to Port Lateral Mark | At a point where a channel divides, when proceeding in the "conventional direction of buoyage", the preferred channel (or primary route) is indicated by a modified starboard-hand lateral mark. |
| 5 | Right-Hand Side of the Waterway | Indicates the right-hand side of the inland waterway. |
| 6 | Left-Hand Side of the Waterway | Indicates the left-hand side of the inland waterway. |
| 7 | Right-Hand Side of the Channel | Indicates the right-hand side of a channel of an inland waterway. |
| 8 | Left-Hand Side of the Channel | Indicates the left-hand side of a channel of an inland waterway. |
| 9 | Bifurcation of the Waterway | Indicates a bifurcation of the inland waterway. |
| 10 | Bifurcation of the Channel | Indicates a bifurcation of a channel of an inland waterway. |
| 11 | Channel Near the Right Bank | Indicates that the channel is near the right bank. |
| 12 | Channel Near the Left Bank | Indicates that the channel is near the left bank. |
| 13 | Channel Cross-Over to the Right Bank | Indicates that the channel crosses from the left to the right bank. |
| 14 | Channel Cross-Over to the Left Bank | Indicates that the channel crosses from the right to the left bank. |
| 15 | Danger Point or Obstacles at the Right-Hand Side | Indicates a danger point or obstacles at the right-hand side. |
| 16 | Danger Point or Obstacles at the Left-Hand Side | Indicates a danger point or obstacles at the left-hand side. |
| 17 | Turn Off at the Right-Hand Side | Indicates a turn off at the right-hand side. |
| 18 | Turn Off at the Left-Hand Side | Indicates a turn off at the left-hand side. |
| 19 | Junction at the Right-Hand Side | Indicates a junction at the right-hand side. |
| 20 | Junction at the Left-Hand Side | Indicates a junction at the left-hand side. |
| 21 | Harbour Entry at the Right-Hand Side | Indicates a harbour entry at the right-hand side. |
| 22 | Harbour Entry at the Left-Hand Side | Indicates a harbour entry at the left-hand side. |
| 23 | Bridge Pier Mark | Indicates a bridge pier in an inland waterway. |
| 24 | Entry From a Lake to a Narrower Waterway, Right Bank | Indicates the right bank of the entry from a lake or a lake-like expansion to a section of the waterway which is narrower. |
| 25 | Entry From a Lake to a Narrower Waterway, Left Bank | Indicates the left bank of the entry from a lake or a lake-like expansion to a section of the waterway which is narrower. |
| 26 | Change Bank | Change bank. |
| 27 | Continue Along Bank | Continue along bank. |

5.98. Category of Cardinal Mark

Definition: The four quadrants (north, east, south and west) are bounded by the true bearings NW-NE, NE-SE, SE-SW and SW-NW taken from the point of interest. A cardinal mark is named after the quadrant in which it is placed. The name of the cardinal mark indicates that it should be passed to the named side of the mark.

CamelCase: categoryOfCardinalMark

Alias: CATCAM

Value type: enumeration

Remarks: Cardinal marks are used in conjunction with the compass to indicate where a mariner will find safe navigable water.Cardinal marks do not have a distinctive shape but are normally pillar or spar. They are always painted in yellow and black horizontal bands and their distinctive double cone top-marks are always black. (Note that such top-marks are encoded as separate TOPMAR objects). Cardinal marks may also have a special system of flashing white lights and if such lights are fitted they are encoded as separate LIGHTS objects.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | North Cardinal Mark | Quadrant bounded by the true bearing NW-NE taken from the point of interest; it should be passed to the north side of the mark. |
| 2 | East Cardinal Mark | Quadrant bounded by the true bearing NE-SE taken from the point of interest. It should be passed to the east side of the mark. |
| 3 | South Cardinal Mark | Quadrant bounded by the true bearing SE-SW taken from the point of interest; it should be passed to the south side of the mark. |
| 4 | West Cardinal Mark | Quadrant bounded by the true bearing SW-NW taken from the point of interest; it should be passed to the west side of the mark. |

5.99. Elevation

Definition: The altitude of the ground level of a feature, measured from a specified vertical datum.

CamelCase: elevation

Alias: ELEVAT

Value type: real

Remarks: No remarks.

5.100. Maximum Display Scale

Definition: The value considered by the Data Producer to be the maximum (largest) scale at which the data is to be displayed before it can be considered to be “grossly overscaled”.

CamelCase: maximumDisplayScale

Alias:

Value type: integer

Remarks: No remarks.

5.101. Minimum Display Scale

Definition: The smallest intended viewing scale for the data.

CamelCase: minimumDisplayScale

Alias:

Value type: integer

Remarks: No remarks.

5.102. Category of Temporal Variation

Definition: An assessment of the likelihood of change over time.

CamelCase: categoryOfTemporalVariation

Alias:

Value type: enumeration

Remarks: No remarks.

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Extreme Event | Indication of the possible impact of a significant event (for example hurricane, earthquake, volcanic eruption, landslide, etc), which is considered likely to have changed the seafloor or landscape significantly. |
| 2 | Likely to Change and Significant Shoaling Expected | Continuous or frequent change (for example river siltation, sand waves, seasonal storms, icebergs, etc) that is likely to result in new significant shoaling. |
| 3 | Likely to Change But Significant Shoaling Not Expected | Continuous or frequent change (for example sand wave shift, seasonal storms, icebergs, etc) that is not likely to result in new significant shoaling. |
| 4 | Likely to Change | Continuous or frequent change to non-bathymetric features (for example river siltation, glacier creep/recession, sand dunes, buoys, marine farms, etc). |
| 5 | Unlikely to Change | Significant change to the seafloor is not expected. |
| 6 | Unassessed | Not having been assessed. |

5.103. Orientation Uncertainty

Definition: The best estimate of the accuracy of a bearing.

CamelCase: orientationUncertainty

Alias:

Value type: real

Remarks: No remarks.

5.104. Horizontal Distance Uncertainty

Definition: The best estimate of the horizontal accuracy of horizontal clearances and distances.

CamelCase: horizontalDistanceUncertainty

Alias: HORACC

Value type: real

Remarks: The error is assumed to be positive and negative. The plus/minus character must not be encoded.

5.105. Change Types

Definition: Different categories or kinds of modifications that can be made to data, positions, or objects. For example, changes may involve updates to position, orientation, or attributes of an object.

CamelCase: changeTypes

Alias:

Value type: enumeration

Remarks: -

Listed Values:

| Code | Label | Definition |
| --- | --- | --- |
| 1 | Advanced notice of changes | - |
| 2 | Discrepancy | - |
| 3 | Proposed changes | - |
| 4 | Temporary changes | - |

6. Complex Attributes

6.1. Change Details

Definition: Specific information or description regarding modifications or updates made to an object, system, or dataset. This term typically includes the nature, scope, and reason for the change, as well as any impact it may have on operations or functionality.

CamelCase: changeDetails

Alias:

Remarks: -

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Aton Commissioning |  | 1 : Buoy establishment 2 : Light establishment 3 : Beacon establishment 4 : Audible signal establishment 5 : Fog signal establishment 6 : AIS transmitter establishment 7 : V-AIS establishment 8 : RACON establishment 9 : DGPS station establishment 10 : eLoran station establishment 11 : DGLONASS station establishment 12 : e-Chayka station establishment 13 : EGNOS station establishment | EN | 0, 1 |
| Aton Removal |  | 1 : Buoy removal 2 : Buoy temporary removal 3 : Light removal 4 : Light temporary removal 5 : Beacon removal 6 : Beacon temporary removal 7 : Fog signal removal 8 : Fog signal temporary removal 9 : Audible signal removal 10 : Audible signal temporary removal 11 : AIS transmitter removal 12 : AIS transmitter temporary removal 13 : V-AIS removal 14 : V-AIS temporary removal 15 : RACON removal 16 : RACON temporary removal 17 : DGPS station removal 18 : DGPS station temporary removal 19 : EGNOS station removal 20 : EGNOS station temporary removal 21 : LORAN station removal 22 : LORAN station temporary removal 24 : eLORAN station temporary removal 25 : Chayka station removal 26 : Chayka station temporary removal 27 : e-Chayka station removal | EN | 0, 1 |
| Aton Replacement |  | 1 : Buoy change 2 : Buoy temporary change 3 : Light change 4 : Light temporary change 5 : Sector light change 6 : Sector light temporary change 7 : Beacon change 8 : Beaon temporary change 9 : Fog signal change 10 : Fog signal temporary chagne 11 : Audible signal change 12 : Audible signal temporary change 13 : V-AIS change 14 : V-AIS temporary change 15 : RACON change 16 : RACON temporary change | EN | 0, 1 |
| Fixed Aton Change |  | 1 : Beacon missing 2 : Beacon damaged 3 : Light beacon Unlit 4 : Light beacon Unreliable 5 : Light beacon Not synchronized 6 : Light beacon damaged 7 : Beacon topmark missing 8 : Beacon topmark damaged 9 : Beacon daymark unreliable 10 : Floodlit beacon Unlit 11 : Beacon restored to normal | EN | 0, 1 |
| Floating Aton Change |  | 1 : Buoy adrift 2 : Buoy damaged 3 : Buoy daymark unreliable 4 : Buoy destroyed 5 : Buoy missing 6 : Buoy move 7 : Buoy off position 8 : Buoy re-establishment 9 : Buoy restored to normal 10 : Buoy topmark damaged 11 : Buoy topmark missing 12 : Buoy will be withdrawn 13 : Buoy withdrawn 14 : Decommissioned for winter 15 : Lifted for Winter 16 : Light buoy Light damaged 17 : Light buoy Light not sychronized 18 : Light buoy Light unlit 19 : Light buoy Light unreliable 20 : Marine Aids to Navigation unreliable 21 : Recommissioned for navigation season 22 : Replaced by Winter Spar 23 : Seasonal decommissioning complete 24 : Seasonal decommissioning in progress 25 : Seasonal recommissioning complete 26 : Seasonal recommissioning in progress | EN | 0, 1 |
| Audible Signal Aton Change |  | 1 : Audible signal out of service 2 : Fog signal out of service 3 : Audible signal operating properly 4 : Fog signal operating properly | EN | 0, 1 |
| Lighted Aton Change |  | 1 : Light unlit 2 : Light unreliable 3 : Light re-establishment 4 : Light range reduced 5 : Light without rhythm 6 : Light out of synchronization 7 : Light daymark unreliable 8 : Light operating properly 9 : Sector light Sector obscured 10 : Front leading/rnage light Unlit 11 : Rear leading/range light Unlit 12 : Front leading/range light Unreliable 13 : Rear leading/range light Unreliable 14 : Front leading/range light Light range reduced 15 : Rear leading/range light Light range reduced 16 : Front leading/range light without rhythm 17 : Rear leading/range light without rhythm 18 : Leading/range lights out of synchronization 19 : Front leading/range beacon Unreliable 20 : Rear leading/range beacon Unreliable 21 : Front leading/range light is operating properly 22 : Rear leading/range light is operating properly 23 : Front leading/range beacon restored to normal 24 : Rear leading/range beacon restored to normal | EN | 0, 1 |

6.2. Obscured Sector

Definition: A portion or sector of a navigational aid, such as a light or beacon, that is blocked or obscured from view due to obstacles (e.g., landforms, buildings, or other structures). In marine or aviation navigation, it usually refers to an area where the light signal or visibility is intentionally or unintentionally reduced or not visible to vessels or aircraft.

CamelCase: obscuredSector

Alias:

Remarks: -

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Sector Limit |  |  | C | 1, 1 |
| Text | (INFORM) (NINFOM) |  | (S) TE | 1, 1 |

6.3. Contact Address

Definition: Direction or superscription of a letter, package, etc., specifying the name of the place to which it is directed, and optionally a contact person or organisation who should receive it.

CamelCase: contactAddress

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Delivery Point |  |  | TE | 0, 1 |
| City Name |  |  | TE | 0, 1 |
| Administrative Division |  |  | TE | 0, 1 |
| Country Name |  |  | TE | 0, 1 |
| Postal Code |  |  | TE | 0, 1 |

6.4. Feature Name

Definition: Provides the name of an entity, defines the national language of the name, and provides the option to display the name at various system display settings.

CamelCase: featureName

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| language |  |  | TE | 1, 1 |
| Name | (OBJNAM) |  | TE | 1, 1 |

6.5. Fixed Date Range

Definition: An active period of a single fixed event or occurrence, as the date range between discrete start and end dates.

CamelCase: fixedDateRange

Alias:

Remarks: Dates must be encoded in the format YYYYMMDD; using 4 digits for the calendar year (YYYY) and, optionally, 2 digits for the month (MM) (for example April = 04) and 2 digits for the day (DD). When no specific month and/or day is required/known, the values are replaced with dashes (-). The date range of a recurring event or occurrence must be encoded using periodicDateRange.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Date End | (DATEND) |  | TD | 0, 1 |
| Date Start | (DATSTA) |  | TD | 0, 1 |

6.6. Horizontal Position Uncertainty

Definition: The best estimate of the accuracy of a position.

CamelCase: horizontalPositionUncertainty

Alias: POSACC

Remarks: The expected input is the maximum of the two-dimensional error. The error is assumed to be positive and negative.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | RE | 1, 1 |
| Uncertainty Variable Factor |  |  | RE | 0, 1 |

6.7. Information

Definition: Textual information about the feature. The information may be provided as a string of text or as a file name of a single external text file that contains the text.

CamelCase: information

Alias: INFORM

Remarks: At least one of the sub-attributes file reference or text must be populated.The sub-attribute file reference is generally used for long text strings or those that require formatting, however, there is no restriction on the type of text (except for lexical level) that can be held in files referenced by sub-attribute file reference.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| File Locator |  |  | TE | 0, 1 |
| File Reference | (TXTDSC) (NTXTDS) |  | TE | 0, 1 |
| Headline |  |  | TE | 0, 1 |
| language |  |  | TE | 1, 1 |
| Text | (INFORM) (NINFOM) |  | TE | 0, 1 |

6.8. Multiplicity of Features

Definition: The number of features of identical character that exist as a co-located group.

CamelCase: multiplicityOfFeatures

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Multiplicity Known |  |  | BO | 1, 1 |
| Number of Features |  |  | IN | 0, 1 |

6.9. Light Sector

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference.

CamelCase: lightSector

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | EN | 1, * |
| Directional Character |  |  | C | 0, 1 |
| Moire Effect |  |  | (S) BO | 0, 1 |
| Orientation |  |  | (S) C | 1, 1 |
| Orientation Uncertainty |  |  | (S) RE | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |
| Sector Arc Extension |  |  | BO | 0, 1 |

6.10. Directional Character

Definition: A directional light is a light illuminating a sector of very narrow angle and intended to mark a direction to follow.

CamelCase: directionalCharacter

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Moire Effect |  |  | BO | 0, 1 |
| Orientation Value | (ORIENT) |  | (S) RE | 1, 1 |

6.11. Orientation

Definition: (1) The angular distance measured from true north to the major axis of the feature. (2) In ECDIS, the mode in which information on the ECDIS is being presented. Typical modes include: north-up - as shown on a nautical chart, north is at the top of the display; Ships head-up - based on the actual heading of the ship, (e.g. Ships gyrocompass); course-up display - based on the course or route being taken.

CamelCase: orientation

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Orientation Uncertainty |  |  | RE | 0, 1 |
| Orientation Value | (ORIENT) |  | RE | 1, 1 |

6.12. Periodic Date Range

Definition: The active period of a recurring event or occurrence.

CamelCase: periodicDateRange

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Date End | (DATEND) |  | TD | 1, 1 |
| Date Start | (DATSTA) |  | TD | 1, 1 |

6.13. Radar Wave Length

Definition: The distance between two successive peaks (or other points of identical phase) on an electromagnetic wave in the radar band of the electromagnetic spectrum.

CamelCase: radarWaveLength

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Radar Band |  |  | TE | 1, 1 |
| Wave Length Value | (RadarWaveLength) |  | RE | 1, 1 |

6.14. Sector Characteristics

Definition: Describes the characteristics of a light sector.

CamelCase: sectorCharacteristics

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing | EN | 1, 1 |
| Light Sector |  |  | C | 1, 10 |
| Colour | (COLOUR) | 1 : White 2 : Black 3 : Red 4 : Green 5 : Blue 6 : Yellow 7 : Grey 8 : Brown 9 : Amber 10 : Violet 11 : Orange 12 : Magenta 13 : Pink | (S) EN | 1, * (ordered) |
| Directional Character |  |  | (S) C | 0, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | (S) EN | 1, 1 |

6.15. Sector Information

Definition: Additional textual information about a light sector.

CamelCase: sectorInformation

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| language |  |  | TE | 0, 1 |
| Text | (INFORM) (NINFOM) |  | TE | 1, 1 |

6.16. Sector Limit

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference. The sector limit specifies the limits of the sector In a clockwise direction around the central feature (for example a light).

CamelCase: sectorLimit

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Sector Limit One | (SECTR1) |  | C | 1, 1 |
| Sector Line Length |  |  | (S) IN | 0, 1 |

6.17. Sector Limit One

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference. Sector limit one specifies the first limit of the sector. The order of sector limit one and sector limit two is clockwise around the central feature (for example a light).

CamelCase: sectorLimitOne

Alias: SECTR1

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Sector Bearing | (SECTR1) (SECTR2) |  | RE | 1, 1 |
| Sector Line Length |  |  | IN | 0, 1 |

6.18. Sector Limit Two

Definition: A sector is the part of a circle between two straight lines drawn from the centre to the circumference. Sector limit two specifies the second limit of the sector. The order of sector limit one and sector limit two is clockwise around the central feature (for example a light).

CamelCase: sectorLimitTwo

Alias: SECTR2

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Sector Bearing | (SECTR1) (SECTR2) |  | RE | 1, 1 |
| Sector Line Length |  |  | IN | 0, 1 |

6.19. Signal Sequence

Definition: The sequence of times occupied by intervals of light/sound and eclipse/silence for all “light characteristics” or sound signals.

CamelCase: signalSequence

Alias: SIGSEQ

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Signal Duration |  |  | RE | 1, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | EN | 1, 1 |

6.20. Spatial Accuracy

Definition: Provides an indication of the vertical and horizontal positional uncertainty of bathymetric data, optionally within a specified date range.

CamelCase: spatialAccuracy

Alias:

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Fixed Date Range |  |  | C | 0, 1 |
| Date End | (DATEND) |  | (S) TD | 0, 1 |
| Uncertainty Variable Factor |  |  | (S) RE | 0, 1 |

6.21. Textual Description

Definition: Encodes the file name of a single external text file that contains the text in a defined language, which provides additional textual information that cannot be provided using other allowable attributes for the feature.

CamelCase: textualDescription

Alias: TXTDSC

Remarks: No remarks.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| File Reference | (TXTDSC) (NTXTDS) |  | TE | 1, 1 (ordered) |
| language |  |  | TE | 0, 1 (ordered) |

6.22. Vertical Uncertainty

Definition: The best estimate of the vertical accuracy of depths, heights, vertical distances and vertical clearances.

CamelCase: verticalUncertainty

Alias: VERACC SOUACC

Remarks: Encodes the vertical uncertainty associated with any vertical measurement.

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Uncertainty Fixed | (POSACC) (SOUACC) (VERACC) |  | RE | 1, 1 |
| Uncertainty Variable Factor |  |  | RE | 0, 1 |

6.23. Rhythm Of Light

Definition: -

CamelCase: rhythmOfLight

Alias:

Remarks: -

SubAttribute Bindings:

| S-10x Attribute | S-57 Acronym | Allowable Encoding Value | Type | Multiplicity |
| --- | --- | --- | --- | --- |
| Light Characteristic | (LITCHR) (Character of Light) | 1 : Fixed 2 : Flashing 3 : Long-Flashing 4 : Quick-Flashing 5 : Very Quick-Flashing 6 : Continuous Ultra Quick-Flashing 7 : Isophased 8 : Occulting 9 : Interrupted Quick Flashing 10 : Interrupted Very Quick Flashing 11 : Interrupted Ultra Quick-Flashing 12 : Morse 13 : Fixed and Flash 14 : Flash and Long-Flash 15 : Occulting and Flash 16 : Fixed and Long-Flash 17 : Occulting Alternating 18 : Long-Flash Alternating 19 : Flash Alternating 20 : Group Alternating 25 : Quick-Flash Plus Long-Flash 26 : Very Quick-Flash Plus Long-Flash 27 : Ultra Quick-Flash Plus Long-Flash 28 : Alternating 29 : Fixed and Alternating Flashing | EN | 1, 1 |
| Signal Group | (SIGGRP) |  | TE | 0, * |
| Signal Period |  |  | RE | 0, 1 |
| Signal Status |  | 1 : Lit/Sound 2 : Eclipsed/Silent | (S) EN | 1, 1 |

7. Roles

7.1. Navigation Line

Definition: -

CamelCase: navigationLine

Alias:

Remarks: No remarks.

7.2. Aton Association By

Definition: -

CamelCase: atonAssociationBy

Alias:

Remarks: No remarks.

7.3. Aton Aggregation By

Definition: -

CamelCase: atonAggregationBy

Alias:

Remarks: No remarks.

7.4. Status Part

Definition: -

CamelCase: Statuspart

Alias:

Remarks: No remarks.

7.5. Physical AIS Broadcasts

Definition: -

CamelCase: physicalAISbroadcasts

Alias:

Remarks: No remarks.

7.6. Virtual AIS Broadcasts

Definition: -

CamelCase: virtualAISbroadcasts

Alias:

Remarks: No remarks.

7.7. Synthetic AIS Broadcasts

Definition: -

CamelCase: syntheticAISbroadcasts

Alias:

Remarks: No remarks.

7.8. Child

Definition: -

CamelCase: child

Alias:

Remarks: No remarks.

7.9. Marking Aton

Definition: -

CamelCase: markingAton

Alias:

Remarks: No remarks.

7.10. Navigable Track

Definition: The role given to the navigable part of the navigation line.

CamelCase: navigableTrack

Alias:

Remarks: No remarks.

7.11. Peer Aton Association

Definition: -

CamelCase: peerAtonAssociation

Alias:

Remarks: No remarks.

7.12. Peer Aton Aggregation

Definition: -

CamelCase: peerAtonAggregation

Alias:

Remarks: No remarks.

7.13. Aton Part

Definition: -

CamelCase: Atonpart

Alias:

Remarks: No remarks.

7.14. Danger

Definition: -

CamelCase: danger

Alias:

Remarks: No remarks.

7.15. Parent

Definition: -

CamelCase: parent

Alias:

Remarks: No remarks.

7.16. Physical AIS Broadcast By

Definition: -

CamelCase: physicalAISbroadcastBy

Alias:

Remarks: No remarks.

7.17. Synthetic AIS Broadcast By

Definition: -

CamelCase: syntheticAISbroadcastBy

Alias:

Remarks: No remarks.

7.18. Virtual AIS Broadcast By

Definition: -

CamelCase: virtualAISbroadcastBy

Alias:

Remarks: No remarks.

8. Information Associations

8.1. Aton Status

Definition:

CamelCase: Atonstatus

Alias:

Remarks: -

Role: Aton Part

Role: Status Part

9. Feature Associations

9.1. Range System

Definition:

CamelCase: RangeSystem

Alias:

Remarks: -

Role: Navigable Track

Role: Navigation Line

9.2. Aton Aggregations

Definition:

CamelCase: AtonAggregations

Alias:

Remarks: -

Role: Peer Aton Aggregation

Role: Aton Aggregation By

9.3. Aton Associations

Definition:

CamelCase: AtonAssociations

Alias:

Remarks: -

Role: Peer Aton Association

Role: Aton Association By

9.4. Dangerous Feature Association

Definition:

CamelCase: DangerousFeatureAssociation

Alias:

Remarks: -

Role: Danger

Role: Marking Aton

9.5. Structure Equipment

Definition:

CamelCase: StructureEquipment

Alias:

Remarks: -

Role: Parent

Role: Child

9.6. Physical AIS

Definition:

CamelCase: PhysicalAIS

Alias:

Remarks: -

Role: Physical AIS Broadcast By

Role: Physical AIS Broadcasts

9.7. Synthetic AIS

Definition:

CamelCase: SyntheticAIS

Alias:

Remarks: -

Role: Synthetic AIS Broadcast By

Role: Synthetic AIS Broadcasts

9.8. Virtual AIS

Definition:

CamelCase: VirtualAIS

Alias:

Remarks: -

Role: Virtual AIS Broadcast By

Role: Virtual AIS Broadcasts